# PFEM / Transolver Work Summary

A plain summary of the completed results (tables and figures) for Timon's round-10 and round-11 feedback, organized against his own feedback points. Full methodology and discussion are in the main report.

## Response to Timon's round-10 feedback (accuracy-matched comparison, throughput, profiling, geometry, break-even)

New since the round-9 reply already sent: (1) a checkpoint-loading bug found and fixed, which had been silently corrupting every round-10 accuracy result with a completely different (data-driven baseline) model; (2) the operator's own accuracy at N=1401 checked directly against real FEM ground truth for the first time, and the coarsest FEM mesh that matches it; (3) a multi-resolution retraining that reduced the error at N=1401 from 44.65% to 5.85% and inverted the peak-stress crossover finding; (4) batch-size/throughput and profiling/torch.compile/TF32 re-verified against both the corrected and the retrained checkpoint, confirming both are checkpoint-independent; (5) a real accuracy-matched break-even. One question remains open and blocks further work: point 4's complex-geometry design (ring with a local notch), already verified with real mesh-convergence evidence, is awaiting Timon's confirmation before any training time is committed to it.

Timon's round-11 feedback (received after the round-10 reply above) asked four further things, all now finished except the last: (1) keep BOTH break-even comparisons, accuracy-matched and resolution-matched -- done, Tables R10-4 and R10-4'; (2) extend the N=1401 accuracy/QoI/break-even analysis to all six geometry-material cases, and clarify in one table which quantity actually defines each case's own coarsest-suitable FEM mesh -- done, Tables R10-6 and R10-7; (3) describe the multi-resolution training protocol in full detail -- done, in the retrain discussion below; (4) train a B1xNeo-Hookean checkpoint directly at N=1401 as an ablation against the zero-shot multi-resolution result -- done, Table R10-5. Point 4 (the complex-geometry example) remains the one open item, exactly as Timon's own instruction was to discuss it separately once points 1-3 above were finished.

Point 1 -- accuracy-matched comparison (operator vs. coarsest suitable FEM), not just matched-resolution.

Answering this required first finding and fixing a real bug: every round-10 script loading the B1 x Neo-Hookean checkpoint tried a storage path that never existed and silently fell back to a completely different (data-driven baseline) model -- caught via a flat, resolution-independent ~620-640% error at every N, the signature of the wrong model rather than a real accuracy failure. Fixed by resolving the checkpoint by content (SHA-256 fingerprint) instead of a hardcoded path.

With the correct checkpoint: operator accuracy at N=1401, checked directly against real FEM ground truth for the first time, is 44.65% relative L2 (39.49% L2-norm, 61.16% H1 semi-norm), degrading smoothly away from the training resolutions (best ~7.4% near N=33-37). Against native FEM at low N (3-49): a near-degenerate mesh already matches or beats the operator's accuracy in L2, H1, energy, and reactions. Peak PK1 stress needs a caveat -- evaluated at a fixed location from a fine reference (fixing an earlier metric-definition mismatch), both methods converge to it very slowly, and the located point sits essentially at a domain-corner boundary-condition transition, a classic near-singularity location in elasticity.

Given the degradation, retrained a new checkpoint on a wider resolution spread (N=21,33,101,201 instead of 21,33 only), using a GPU-accelerated data path verified bit-identical to the original solver first. Result: the monotonic degradation is gone. Error drops from 44.65% to 5.85% at N=1401 (7.6x), and instead of climbing indefinitely, plateaus around 5.8-5.9% beyond N~50.

| N | Original checkpoint (N=21,33 only) | Retrained (N=21,33,101,201) |
| --- | --- | --- |
| 13 | 13.85% | 11.74% |
| 33 | 7.37% | 3.50% |
| 49 | 8.50% | 2.32% |
| 101 | 14.99% | 3.55% |
| 201 | 22.28% | 4.85% |
| 701 | 34.34% | 5.88% |
| 1401 | 44.65% | 5.85% |

Table R10-1. disp_rel_L2 vs. real FEM ground truth, original vs. retrained checkpoint.

Figure R10-A. Original checkpoint's accuracy degradation, N=49 to N=1401 (pre-retraining).

The retrained checkpoint also flips the peak-stress finding: its peak-stress accuracy (42-70%, improved at every N) is now BETTER than anything FEM achieves in the whole low-N range (62% at best) for every operator resolution N>=29 -- FEM would need N>49 (untested) to match it. The "coarsest suitable FEM" is now bound by tangent energy, not peak stress, and dropped from N=17-45 (original checkpoint) to a near-degenerate N=9-17 (retrained checkpoint) for almost every resolution. Same domain-corner caveat still applies to peak stress specifically -- the improvement itself is real and independently verified, not a training-validation artifact.

The same fix (retrain on N=21,33,101,201 instead of N=21,33 only) was since extended to every other case with the same degradation signature. Round-11 point 3 (multi-resolution training protocol, in full detail, as requested): weighting is equal by construction (400 train / 100 val samples per resolution, identical count at all four); each batch is homogeneous in one resolution, but the full batch list spanning all four is shuffled together so different resolutions interleave randomly through the epoch; the loss carries no explicit cross-resolution weight. N=101/201 were added because they were exactly where the degradation sweep had already measured real error (15.0%/22.3%) before any retraining ran. Two methods changes made the later retrains cheaper: TF32 training (verified safe, 2.08x at N=201, the resolution it actually helps), and a real bug found and fixed this session -- the GPU fast data-generation path had only ever been wired up for B1, so B2's own --fast_solver/--nsteps did nothing at all until now; every earlier B2 job silently ran on the slow CPU solver regardless of its own launch command.

Results, N=1401, old vs. new checkpoint (Table R10-2): B1 x Mooney-Rivlin 39.20% -> 15.04% (~62% relative reduction); B1 x Arruda-Boyce 45.62% -> 25.05% (~45%, and its own real GPU run is reassuring evidence that a shared chain-locking-clamp exposure with this project's own torch-fem Arruda-Boyce runs does not bite in practice here -- all 16 resolutions, both checkpoints, converged cleanly); B2 x Mooney-Rivlin 49.47% -> 13.10% (~73.5%, the largest reduction so far, and the clearest case of a genuinely FLAT error across N=37-1401 rather than merely a smaller one at N=1401); B2 x Neo-Hookean 46.37% -> 36.05% (~22.3%, the smallest of the five reductions -- unlike Mooney-Rivlin's flat curve, this checkpoint's error is lowest right where it was trained, N=101 at 12.98%, and rises again beyond that to 36-40% by N=1401, still clearly better than the original checkpoint's 46.4-46.7% there).

Round-11 point 4 -- direct-N1401 training ablation -- has since completed. A B1 x Neo-Hookean checkpoint trained DIRECTLY at N=1401 (100 samples, early stopped at epoch 36, best epoch 20) is cheaper to train (8.00h vs. 11.63h, ~30% less GPU time) but 6.3x LESS accurate (36.65% vs. 5.85% disp_rel_L2) than the multi-resolution zero-shot checkpoint, with essentially identical inference cost (2318.8 ms vs. 2292.1 ms/sample). A genuine result in favour of the multi-resolution strategy: not just "it also works zero-shot," but "it produces a substantially better model for less than 50% more training time." Table R10-5.

| Metric | Direct @N=1401 | Multi-res (zero-shot) |
| --- | --- | --- |
| Training wall-clock | 8.00 h | 11.63 h |
| disp_rel_L2 @ N=1401 | 36.65% | 5.85% |
| Inference (eager fp32) | 2318.8 ms | 2292.1 ms |

Table R10-5. Direct-N1401 training ablation vs. multi-resolution zero-shot, B1 x Neo-Hookean.

Figure R10-E. Direct-N1401 training vs. multi-resolution zero-shot, training cost and accuracy (Table R10-5).

| Case | Old (N=1401) | New (N=1401) | Relative reduction |
| --- | --- | --- | --- |
| B1 x Neo-Hookean | 44.65% | 5.85% | ~87% (7.6x) |
| B1 x Mooney-Rivlin | 39.20% | 15.04% | ~62% |
| B1 x Arruda-Boyce | 45.62% | 25.05% | ~45% |
| B2 x Mooney-Rivlin | 49.47% | 13.10% | ~73.5% |
| B2 x Neo-Hookean | 46.37% | 36.05% | ~22.3% |

Table R10-2. Multi-resolution retrain fix, every completed case, disp_rel_L2 at N=1401.

Figure R10-B. Multi-resolution retrain fix, old vs. new checkpoint, all completed cases (Table R10-2).

Point 2 -- batch size and throughput at matched GPU memory.

| Scenario | Max batch size | Peak GPU memory | Throughput |
| --- | --- | --- | --- |
| Operator (own ceiling) | 8,192 | 47.6 GB | 4,203.79 samples/s |
| FEM (own ceiling) | 256 | 38.7 GB | 2.86 samples/s |
| FEM (capped at operator's 47.6 GB) | 256 | 38.7 GB | 2.86 samples/s |
| Operator (capped at FEM's 38.7 GB) | 4,096 | 23.8 GB | 4,207.75 samples/s |

Table R10-2. Max feasible batch size and throughput, N=21.

Figure R10-B. Throughput and peak memory vs. batch size, operator vs. FEM, own memory ceilings.

Peak throughput favors the operator ~1,470x regardless of memory cap -- FEM's own ceiling is already below saturation, while the operator barely gives up anything capped down to FEM's much smaller budget. Batching helps the operator ~20x (bs=1 to its own ceiling) while FEM barely benefits (~5x), matching Timon's own stated expectation. Re-measured against both the corrected and the retrained checkpoint: max-batch-size throughput 4,182.27 vs. 4,203.79 samples/s (0.5% relative difference) -- checkpoint-independent, as expected for a timing measurement.

Point 3 -- NO inference optimization and profiling.

Pure GPU forward-pass time, fp32, batch size 1, N=1401: 2,290 ms/sample. Peak memory 23.16 GB, higher than the GPU-FEM solver's own peak at the same N (15.6-20.5 GB). Profiler: ~67% in batched matmuls, ~22% in MLP layers -- genuine core compute, not an obvious inefficiency.

| Variant | ms/sample | Speedup vs. eager | Output diff vs. eager |
| --- | --- | --- | --- |
| Eager (fp32) | 2,292.1 | 1.00x | -- |
| torch.compile | 2,145.1 | 1.07x | 6.0e-7 |
| Eager + TF32 | 491.2 | 4.67x | 3.2e-3 |
| Compile + TF32 | 394.0 | 5.82x | 3.2e-3 |

Table R10-3. Inference-speed optimization attempts, N=1401, correctness-checked against strict fp32 eager.

Figure R10-C. Same data as Table R10-3.

torch.compile alone: 1.07x (correctness-checked). TF32 matmul precision: 4.67x alone, 5.82x combined with torch.compile -- the best result found, at a real but small accuracy cost (~3.2e-3 relative), checked directly. Re-measured against both the corrected and the retrained checkpoint: all four variants within 0.01-0.8% of the original numbers (eager 2,286.9 vs. 2,292.1 ms; compile 2,145.2 vs. 2,145.1 ms; eager+TF32 487.3 vs. 491.2 ms; compile+TF32 395.0 vs. 394.0 ms) -- checkpoint-independent as expected.

Point 4 -- complex-geometry example, one question before committing training time.

Designed and verified a candidate before spending GPU-hours training: extending the pressure-vessel-like ring case with a single smooth local notch (Gaussian dimple) in the inner wall -- still simply connected, reusing existing solver machinery. Mesh-convergence check across four resolutions confirms a genuine local stress concentration: peak stress at the notch still rising (11.70 -> 13.94, +3.7% at the finest mesh, still rising) while global displacement is essentially flat (+0.27%) -- exactly the signature Timon described of a globally-converged field hiding an under-resolved local quantity. Awaiting his confirmation of this design before training a full model on it.

Point 5 -- break-even analysis, accuracy-matched.

Using the retrained checkpoint's own coarsest-suitable FEM at N=1401 (N=11, bound by tangent energy per the crossover above) against the operator's real inference cost there, and the real training wall-clock for the retrained checkpoint (41,881 s, ~11.6h -- confirmed from its own training log; it stopped itself via early stopping, 8 checks with no improvement, exactly as configured):

| NO inference mode | ms/sample @ N=1401 | vs. FEM@N=11 (1,625.6 ms) | Break-even |
| --- | --- | --- | --- |
| Eager fp32 (default) | 2,292.1 | 0.71x (slower) | Never -- accuracy-matched FEM already cheaper |
| Compile + TF32 | 394.0 | 4.13x (faster) | After 34,005 samples (~3.72 GPU-hours) |

Table R10-4. Accuracy-matched break-even: operator@N=1401 vs. FEM@N=11. Distinct from the earlier matched-resolution break-even (Point 3 above): this one intentionally runs the two methods at different N, matched by accuracy rather than by mesh.

This is the one place in the whole round-10 investigation where the operator's default (unoptimized) mode does not come out ahead: run naively, it never repays its own training cost against a FEM mesh that is only as accurate as it needs to be. With the point-3 optimizations (torch.compile + TF32), it both wins per-sample (4.13x) and repays its training cost cheaply -- about 3.72 GPU-hours against 11.6 GPU-hours of training. The operator's economic case depends on deploying it with these optimizations, not on its default settings -- reported honestly rather than only the favorable case.

Round-11 point 1 (keep BOTH break-even comparisons): the accuracy-matched one above (Table R10-4) is one half; the advisor asked to also keep a SECOND comparison, both methods at the SAME resolution N=1401 rather than each at its own accuracy-matched mesh -- expected, correctly, to look much more favourable to the operator. Table R10-4', all six cases, operator's own default eager fp32 forward pass at N=1401 vs. torch-fem's own real GPU solve time at the same N.

| Case | torch-fem @N=1401 | Operator @N=1401 | Speedup | Break-even |
| --- | --- | --- | --- | --- |
| B1 x Neo-Hookean | 135.00 s | 2292.1 ms | 58.9x | 316 samples |
| B1 x Mooney-Rivlin | 133.92 s | 2361.9 ms | 56.7x | training cost unknown |
| B1 x Arruda-Boyce | FAILED (see note) | - | - | - |
| B2 x Neo-Hookean | 205.14 s | 2351.1 ms | 87.3x | training cost unknown |
| B2 x Mooney-Rivlin | 207.21 s | 2356.0 ms | 88.0x | training cost unknown |
| B2 x Arruda-Boyce | FAILED (see note) | - | - | - |

Table R10-4'. Resolution-matched break-even, both methods at N=1401, all six cases. At matched resolution the operator wins by 57-89x even unoptimized -- unlike Table R10-4's own accuracy-matched comparison, where the default mode does not break even at all. All four non-Arruda-Boyce numbers are from a single clean re-run (2026-09-17) after fixing a real memory-cleanup bug in the sweep script (an exception-chain reference kept a failed case's own GPU tensors pinned into the next case) -- this re-run succeeded for every non-Arruda-Boyce case in one pass, confirming the fix on real GPU. Both Arruda-Boyce cases fail genuinely and consistently across every run: torch-fem's own Newton-Raphson solve does not converge at N=1401, root-caused to a real CUDA OOM inside torch-fem's own Hessian assembly for this specific material.

Figure R10-D. Resolution-matched speedup, all six cases (Table R10-4'). (Figure R10-C already exists for Table R10-3's own data.)

Round-11 point 2, part A (extend the N=1401 QoI analysis to all remaining cases): task #22's peak-stress QoI, the five cases beyond B1xNeo-Hookean (Table R10-6). Caveat: B1xMooney-Rivlin, B1xArruda-Boyce, B2xMooney-Rivlin and B2xNeo-Hookean here reflect their ORIGINAL pre-retrain checkpoints (their disp_rel_L2 matches the "OLD" column in Table R10-2 exactly) -- the peak-stress metric has not been recomputed against this week's new retrained checkpoints for those four. B2xArruda-Boyce has only one checkpoint, no caveat.

| Case | disp_rel_L2 @N1401 | peak_stress_rel_err @N1401 |
| --- | --- | --- |
| B1 x Mooney-Rivlin | 39.20% | 78.05% (orig. checkpoint) |
| B1 x Arruda-Boyce | 45.62% | 78.25% (orig. checkpoint) |
| B2 x Neo-Hookean | 46.37% | 49.48% (orig. checkpoint) |
| B2 x Mooney-Rivlin | 49.47% | 48.13% (orig. checkpoint) |
| B2 x Arruda-Boyce | 39.73% | 39.97% |

Table R10-6. Peak-stress QoI at N=1401, five cases. B1's own two rows (78.05%, 78.25%) are structurally worse than every B2 case (39.97%-49.48%), consistent with B1's own boundary-corner stress-singularity caveat noted above. B2xArruda-Boyce (39.97%) has the BEST peak-stress accuracy of all six cases despite failing the break-even above -- unrelated findings: that failure is torch-fem's own reference solve running out of memory, not a property of the operator's own accuracy.

Figure R10-F. Peak-stress QoI vs. displacement error, five cases (Table R10-6).

Round-11 point 2, part B (clarify, in one table, which quantity actually defines each case's own coarsest-suitable FEM mesh): does the same QoI determine the crossover every time? Already answered for B1xNeo-Hookean (N=11, bound by tangent energy) -- repeated here for the other five cases (Table R10-7). Checkpoint caveat as above: B1xMooney-Rivlin, B1xArruda-Boyce, B2xMooney-Rivlin and B2xNeo-Hookean reflect their ORIGINAL pre-retrain checkpoints here (task #22 predates all four retrains).

| Case | Coarsest suitable N | Binding QoI |
| --- | --- | --- |
| B1 x Mooney-Rivlin | 3 | all five metrics at once (L2, H1, energy, peak stress, reaction) |
| B1 x Arruda-Boyce | 3 | all five metrics at once (L2, H1, energy, peak stress, reaction) |
| B2 x Neo-Hookean | 4 | tangent energy |
| B2 x Mooney-Rivlin | 4 | tangent energy |
| B2 x Arruda-Boyce | 4 | tangent energy |

Table R10-7. Coarsest suitable FEM and its binding QoI, five cases, N=1401. N=3 is the coarsest resolution tested, so the two B1 rows' true crossover may lie even coarser.

Mixed answer: all three B2 materials share the SAME binding metric (tangent energy), matching B1xNeo-Hookean -- but B1's other two materials cross on EVERY metric simultaneously, at the coarsest mesh tested (N=3), because the pre-retrain operator's own accuracy there is poor enough (L2_rel 33.65% Mooney-Rivlin, 44.57% Arruda-Boyce) that even the cheapest FEM mesh in this report already wins on every quantity of interest.