PFEM / Transolver Work Summary -- Round 13

This round covers items 1 and 2 of the advisor's follow-up email after round 12 ("let's wrap up the benchmark work"). Item 3 (drop IGA/NURBS) needed no report content -- confirmed explicit future work only, discussed after this paper. Item 4 (a new, harder 3D example) is its own much larger deliverable and will get its own round and Summary once built, per the advisor's own stated order. As with round 12, everything below is copied verbatim (same tables, same text) out of the now-updated cumulative Report (PFEM_Transolver_Report_2026-09-19b.docx) -- nothing summarized or re-derived.

Item 1 -- QoI-accuracy-threshold vs. required-FEM-resolution vs. break-even

“Finalize the low-N accuracy comparison (drop N=1401 for B1/B2) and add a table showing, for a few QoI-accuracy thresholds, the required FEM resolution and the break-even point.” — Timon

Round-13 (the advisor's follow-up email after round 12): finalize the low-N comparison and add one new analysis -- for a few QoI-accuracy thresholds (1%, 2%, 5%, the advisor's own examples), per QoI, the minimum GPU-native-FEM resolution N required to reach it, and the operator's own break-even point there. This reuses round 12's own consistent-field accuracy data (FEM and the operator on the identical field, scored against one real fine reference) for the accuracy side, and adds one new real measurement for the cost side: our own GPU-native FEM solver's (gpu_fem_solver.py) per-sample wall-clock at every one of the sixteen LOW_N resolutions, not just the single N=11 point the earlier accuracy-matched break-even table (Table 18-R10e) used. Break-even always uses the operator's own compile+TF32 cost at N=1401 (its real deployment resolution, essentially case-independent at ~394ms -- see Table 18-R10h) against that resolution's own training wall-clock -- the SAME operator baseline as the existing break-even tables, not a new one. One table per case below.

B1 x Neo-Hookean:

Table 18-R11a. B1 x Neo-Hookean: minimum GPU-native-FEM resolution N required to reach each QoI-accuracy threshold (FEM's own error, from Table 18-R10o/18-R10p), that resolution's own real per-sample GPU-FEM cost, and the operator's break-even point against it (training wall-clock divided by the per-sample saving vs. the operator's own compile+TF32 cost at N=1401, its real deployment resolution).

| QoI | Threshold | Required N | FEM achieved | FEM ms/sample | Break-even |
| --- | --- | --- | --- | --- | --- |
| Displacement L2 | 1% | 9 | 0.81% | 1659.8 ms | 33089 samples |
| Displacement L2 | 2% | 6 | 1.64% | 1669.9 ms | 32828 samples |
| Displacement L2 | 5% | 4 | 3.63% | 1654.0 ms | 33241 samples |
| H1 semi-norm | 1% | n/a | n/a | n/a | not reached by N=49 |
| H1 semi-norm | 2% | 41 | 1.56% | 6506.4 ms | 6852 samples |
| H1 semi-norm | 5% | 17 | 4.36% | 1700.3 ms | 32063 samples |
| Tangent energy | 1% | n/a | n/a | n/a | not reached by N=49 |
| Tangent energy | 2% | 37 | 1.91% | 4588.9 ms | 9984 samples |
| Tangent energy | 5% | 13 | 4.63% | 1672.0 ms | 32773 samples |
| Reaction force (B1 only) | 1% | 9 | 0.84% | 1659.8 ms | 33089 samples |
| Reaction force (B1 only) | 2% | 9 | 0.84% | 1659.8 ms | 33089 samples |
| Reaction force (B1 only) | 5% | 5 | 3.39% | 1666.6 ms | 32911 samples |
| Region-Cauchy avg | 1% | 21 | 0.92% | 1736.9 ms | 31189 samples |
| Region-Cauchy avg | 2% | 11 | 1.73% | 1660.7 ms | 33065 samples |
| Region-Cauchy avg | 5% | 3 | 3.86% | 1636.0 ms | 33723 samples |

B1 x Mooney-Rivlin:

Table 18-R11b. B1 x Mooney-Rivlin: minimum GPU-native-FEM resolution N required to reach each QoI-accuracy threshold (FEM's own error, from Table 18-R10q/18-R10r), that resolution's own real per-sample GPU-FEM cost, and the operator's break-even point against it (training wall-clock divided by the per-sample saving vs. the operator's own compile+TF32 cost at N=1401, its real deployment resolution).

| QoI | Threshold | Required N | FEM achieved | FEM ms/sample | Break-even |
| --- | --- | --- | --- | --- | --- |
| Displacement L2 | 1% | 9 | 0.80% | 1936.0 ms | 34580 samples |
| Displacement L2 | 2% | 6 | 1.61% | 1938.4 ms | 34525 samples |
| Displacement L2 | 5% | 4 | 3.51% | 1922.5 ms | 34886 samples |
| H1 semi-norm | 1% | n/a | n/a | n/a | not reached by N=49 |
| H1 semi-norm | 2% | 41 | 1.60% | 6610.0 ms | 8577 samples |
| H1 semi-norm | 5% | 17 | 4.43% | 1994.5 ms | 33317 samples |
| Tangent energy | 1% | n/a | n/a | n/a | not reached by N=49 |
| Tangent energy | 2% | 37 | 1.97% | 4679.5 ms | 12442 samples |
| Tangent energy | 5% | 13 | 4.71% | 1969.6 ms | 33843 samples |
| Reaction force (B1 only) | 1% | 9 | 0.72% | 1936.0 ms | 34580 samples |
| Reaction force (B1 only) | 2% | 6 | 1.81% | 1938.4 ms | 34525 samples |
| Reaction force (B1 only) | 5% | 4 | 4.86% | 1922.5 ms | 34886 samples |
| Region-Cauchy avg | 1% | 25 | 0.70% | 2099.0 ms | 31274 samples |
| Region-Cauchy avg | 2% | 11 | 2.00% | 1952.0 ms | 34225 samples |
| Region-Cauchy avg | 5% | 9 | 4.19% | 1936.0 ms | 34580 samples |

B1 x Arruda-Boyce:

Table 18-R11c. B1 x Arruda-Boyce: minimum GPU-native-FEM resolution N required to reach each QoI-accuracy threshold (FEM's own error, from Table 18-R10s/18-R10t), that resolution's own real per-sample GPU-FEM cost, and the operator's break-even point against it (training wall-clock divided by the per-sample saving vs. the operator's own compile+TF32 cost at N=1401, its real deployment resolution).

| QoI | Threshold | Required N | FEM achieved | FEM ms/sample | Break-even |
| --- | --- | --- | --- | --- | --- |
| Displacement L2 | 1% | 9 | 0.62% | 2637.9 ms | 17377 samples |
| Displacement L2 | 2% | 5 | 1.88% | 2585.9 ms | 17789 samples |
| Displacement L2 | 5% | 4 | 3.05% | 2596.5 ms | 17704 samples |
| H1 semi-norm | 1% | n/a | n/a | n/a | not reached by N=49 |
| H1 semi-norm | 2% | 33 | 1.98% | 3522.2 ms | 12464 samples |
| H1 semi-norm | 5% | 13 | 4.67% | 2677.1 ms | 17078 samples |
| Tangent energy | 1% | n/a | n/a | n/a | not reached by N=49 |
| Tangent energy | 2% | 29 | 1.90% | 2822.4 ms | 16056 samples |
| Tangent energy | 5% | 11 | 4.56% | 2636.1 ms | 17391 samples |
| Reaction force (B1 only) | 1% | 9 | 0.81% | 2637.9 ms | 17377 samples |
| Reaction force (B1 only) | 2% | 9 | 0.81% | 2637.9 ms | 17377 samples |
| Reaction force (B1 only) | 5% | 5 | 3.36% | 2585.9 ms | 17789 samples |
| Region-Cauchy avg | 1% | 4 | 0.93% | 2596.5 ms | 17704 samples |
| Region-Cauchy avg | 2% | 3 | 1.83% | 2590.9 ms | 17748 samples |
| Region-Cauchy avg | 5% | 3 | 1.83% | 2590.9 ms | 17748 samples |

B2 x Neo-Hookean:

Table 18-R11d. B2 x Neo-Hookean: minimum GPU-native-FEM resolution N required to reach each QoI-accuracy threshold (FEM's own error, from Table 18-R10u/18-R10v), that resolution's own real per-sample GPU-FEM cost, and the operator's break-even point against it (training wall-clock divided by the per-sample saving vs. the operator's own compile+TF32 cost at N=1401, its real deployment resolution). Reaction force is n/a (no reaction resultant defined for the B2 ring geometry).

| QoI | Threshold | Required N | FEM achieved | FEM ms/sample | Break-even |
| --- | --- | --- | --- | --- | --- |
| Displacement L2 | 1% | 13 | 0.70% | 1710.3 ms | 30053 samples |
| Displacement L2 | 2% | 9 | 1.54% | 1669.4 ms | 31016 samples |
| Displacement L2 | 5% | 6 | 3.69% | 1652.1 ms | 31442 samples |
| H1 semi-norm | 1% | n/a | n/a | n/a | not reached by N=49 |
| H1 semi-norm | 2% | 41 | 1.37% | 6524.0 ms | 6454 samples |
| H1 semi-norm | 5% | 17 | 4.89% | 1728.9 ms | 29633 samples |
| Tangent energy | 1% | n/a | n/a | n/a | not reached by N=49 |
| Tangent energy | 2% | 33 | 1.77% | 3199.5 ms | 14101 samples |
| Tangent energy | 5% | 17 | 4.04% | 1728.9 ms | 29633 samples |
| Region-Cauchy avg | 1% | 13 | 0.83% | 1710.3 ms | 30053 samples |
| Region-Cauchy avg | 2% | 9 | 1.63% | 1669.4 ms | 31016 samples |
| Region-Cauchy avg | 5% | 6 | 4.20% | 1652.1 ms | 31442 samples |

B2 x Mooney-Rivlin:

Table 18-R11e. B2 x Mooney-Rivlin: minimum GPU-native-FEM resolution N required to reach each QoI-accuracy threshold (FEM's own error, from Table 18-R10w/18-R10x), that resolution's own real per-sample GPU-FEM cost, and the operator's break-even point against it (training wall-clock divided by the per-sample saving vs. the operator's own compile+TF32 cost at N=1401, its real deployment resolution). Reaction force is n/a (no reaction resultant defined for the B2 ring geometry).

| QoI | Threshold | Required N | FEM achieved | FEM ms/sample | Break-even |
| --- | --- | --- | --- | --- | --- |
| Displacement L2 | 1% | 11 | 0.97% | 1985.7 ms | 3982 samples |
| Displacement L2 | 2% | 9 | 1.49% | 1985.0 ms | 3983 samples |
| Displacement L2 | 5% | 6 | 3.57% | 1961.3 ms | 4043 samples |
| H1 semi-norm | 1% | n/a | n/a | n/a | not reached by N=49 |
| H1 semi-norm | 2% | 41 | 1.37% | 6630.8 ms | 1016 samples |
| H1 semi-norm | 5% | 17 | 4.89% | 2040.4 ms | 3849 samples |
| Tangent energy | 1% | n/a | n/a | n/a | not reached by N=49 |
| Tangent energy | 2% | 33 | 1.80% | 3243.9 ms | 2223 samples |
| Tangent energy | 5% | 17 | 4.12% | 2040.4 ms | 3849 samples |
| Region-Cauchy avg | 1% | 13 | 0.72% | 1996.0 ms | 3956 samples |
| Region-Cauchy avg | 2% | 9 | 1.51% | 1985.0 ms | 3983 samples |
| Region-Cauchy avg | 5% | 6 | 3.91% | 1961.3 ms | 4043 samples |

B2 x Arruda-Boyce:

Table 18-R11f. B2 x Arruda-Boyce: minimum GPU-native-FEM resolution N required to reach each QoI-accuracy threshold (FEM's own error, from Table 18-R10y/18-R10z), that resolution's own real per-sample GPU-FEM cost, and the operator's break-even point against it (training wall-clock divided by the per-sample saving vs. the operator's own compile+TF32 cost at N=1401, its real deployment resolution). Reaction force is n/a (no reaction resultant defined for the B2 ring geometry).

| QoI | Threshold | Required N | FEM achieved | FEM ms/sample | Break-even |
| --- | --- | --- | --- | --- | --- |
| Displacement L2 | 1% | 13 | 0.73% | 2652.3 ms | 3572 samples |
| Displacement L2 | 2% | 9 | 1.59% | 2648.4 ms | 3578 samples |
| Displacement L2 | 5% | 6 | 3.78% | 2593.3 ms | 3668 samples |
| H1 semi-norm | 1% | n/a | n/a | n/a | not reached by N=49 |
| H1 semi-norm | 2% | 41 | 1.37% | 7091.9 ms | 1204 samples |
| H1 semi-norm | 5% | 17 | 4.89% | 2666.0 ms | 3551 samples |
| Tangent energy | 1% | n/a | n/a | n/a | not reached by N=49 |
| Tangent energy | 2% | 33 | 1.74% | 3545.3 ms | 2560 samples |
| Tangent energy | 5% | 17 | 3.92% | 2666.0 ms | 3551 samples |
| Region-Cauchy avg | 1% | 13 | 0.98% | 2652.3 ms | 3572 samples |
| Region-Cauchy avg | 2% | 9 | 1.81% | 2648.4 ms | 3578 samples |
| Region-Cauchy avg | 5% | 6 | 4.72% | 2593.3 ms | 3668 samples |

Two findings worth stating plainly. First, across all six cases and every threshold tested, the required FEM resolution never exceeds N=41, and its own per-sample cost there (up to ~7.1s) never comes close to the operator's own ~394ms -- FEM never becomes cheaper than the operator at any accuracy level tested, so a break-even point always exists and is always finite; the six cases' break-even sample counts differ almost entirely by each case's own training cost, not by its accuracy numbers (compare B1xMooney-Rivlin's ~34,500 samples, driven by its 14.81h training run, against B2xMooney-Rivlin's ~4,000 samples, driven by its own much cheaper 1.76h run, at the SAME L2@2% threshold). Second, H1 semi-norm and tangent energy never reach the 1% threshold anywhere in the tested range (N=3..49) for any of the six cases -- consistent with this project's own repeated finding that energy/H1-type norms converge more slowly than displacement L2, and a real, disclosed gap: a 1% energy/H1 target would need FEM resolutions finer than this sweep covers, not a number this table can currently supply.

Item 2 -- confirm the GPU-native solver as the primary timing baseline

“Use your most efficient validated GPU-native FEM solver as the primary timing baseline; keep torch-fem/TensorMesh as secondary.” — Timon

Investigated via direct code inspection, not assumption: Table 18-R10e's own finite-element measurement already comes from gpu_fem_benchmark.py -> gpu_fem_solver.py -- our own GPU-native Total-Lagrangian Newton solver, built earlier per the advisor's own request for a GPU-native comparison, completely independent of torch-fem. Torch-fem is used only in the separate resolution-matched table (18-R10e'), matching the advisor's own request to keep that comparison apart. So this item needed no computation change -- only making it explicit in the Report text, since the cell producing this number only named it "torch-fem" in its own informal print() statements, never in the actual Report wording. Two paragraphs (below) were updated accordingly; nothing else changed.

Point 5 asked for an updated break-even analysis. Using the retrained checkpoint's own coarsest-suitable finite-element mesh at N=1401 (N=11, bound by the tangent-energy norm per the crossover above) against the operator's own real inference cost there, and the real training wall-clock for the retrained checkpoint (41,881 s, about 11.6 hours, confirmed above): in its default eager fp32 deployment mode, the operator never recovers its own training cost against this baseline -- the accuracy-matched finite-element mesh is already cheaper per sample (1,625.6 ms) than the operator's default forward pass (2,292.1 ms). With the torch.compile plus TF32 optimization above (394.0 ms/sample), the operator is instead 4.13x faster per sample and recovers its training cost after 34,005 solved problems, about 3.72 GPU-hours of inference against the 11.6 GPU-hours spent training. This is the only point in the whole round-10 investigation where the operator's default, unoptimized deployment mode does not come out ahead; its economic case against a genuinely accuracy-matched baseline rests on deploying it with the optimizations found under point 3, not on its default settings. (The finite-element side of this comparison is our own GPU-native Total-Lagrangian Newton solver, gpu_fem_solver.py -- built earlier per the advisor's own request for a GPU-native comparison, and already the primary timing baseline used here; torch-fem appears only in the separate resolution-matched comparison below, Table 18-R10e', which intentionally asks a different question -- same-N, not accuracy-matched -- per the advisor's own request to keep the two comparisons apart.)

| NO inference mode | ms/sample @ N=1401 | vs. FEM@N=11 (1,625.6 ms) | Break-even |
| --- | --- | --- | --- |
| Eager fp32 (default) | 2,292.1 | 0.71x (slower) | Never -- accuracy-matched FEM already cheaper |
| Compile + TF32 | 394.0 | 4.13x (faster) | After 34,005 samples (~3.72 GPU-hours) |

Table 18-R10e. Accuracy-matched break-even: operator@N=1401 vs. our own GPU-native finite-element solver (gpu_fem_solver.py) @N=11 (its coarsest suitable mesh for the retrained checkpoint). Distinct from the matched-resolution/matched-batch-size break-even of Section 8.4: this comparison intentionally runs the two methods at different N, matched by accuracy rather than by mesh.