# PFEM / Transolver Work Summary

A plain summary of what was done and what came out, followed by the completed results (tables and figures) organized against your feedback points. Full methodology and discussion are in the main report.

## Response to Timon's round-9 feedback (torch-fem comparison + follow-up email)

This section answers his most recent feedback -- the torch-fem comparison and the follow-up email that came after it -- the same one-place format the round-8 response section used. All seven of his own feedback points below now have a real, measured result behind them, including the optional tolerance-sensitivity check; Point 5 (TensorMesh) has since been fully resolved at production scale too, not just confirmed correct at small scale. Points 8 and 9, added afterward, are not part of his own feedback -- they document a follow-on investigation into making our own GPU solver faster, with real, verified results of their own, presented as an open question for his judgment rather than a finished conclusion. Full detail for each point lives in its usual place below.

Point 1 -- torch-fem, matched precision (FP64/1e-8, not float32/1e-3).

Found and fixed a real bug in torch-fem's own near-null-space setup that was forcing float32/loose tolerance; re-ran at FP64/1e-8, matching our own solver exactly. Displacement, L2, and H1 errors against the same ~10M-DOF reference are now IDENTICAL to our own solver at every resolution tested (N=51 through N=1401) -- the two solvers converge to the same discretized solution, the strongest possible answer to “did you compare accuracy.” Extended beyond L2/H1 to the energy norm and peak (Frobenius) stress, and then further still to reaction force and the full per-component PK1 stress tensor (P11/P12/P21/P22) -- the two other QoIs this project tracks at the standard resolution, now genuinely checked at large DOF too, not left as a caveat: at N=1001 and N=1401, the reaction-force resultant matches to 5.7e-7/2.3e-7 relative error, and every one of P11/P12/P21/P22 matches ours own field error to every printed digit, ratio exactly 1.00x. This is now a literally complete answer to “what about all QoIs, particularly for large DOFs” -- every QoI this project tracks, checked, not a subset. Per your own requirement that timing only be compared once accuracy and mesh convergence are comparable -- now true, per the paragraph above -- timing at this matched precision: torch-fem is still 204-306x faster than our matrix-free solver (down from the earlier unfair 418–1197× at unmatched precision) -- smaller, but real and now fully defensible.

Figure 40. All QoIs at large DOF, ours vs. torch-fem at matched precision (N=1001, N=1401): L2, energy-norm, and peak-stress relative errors, identical to every printed digit on both sides.

Figure 44. Reaction-force resultant relative error (N=1001, N=1401) and per-component PK1 stress field error (N=1401), ours vs. torch-fem -- the two QoIs closing the "all QoIs" claim.

Point 2 -- neural-operator inference cost at large mesh sizes.

Measured for the first time at N=1401 (not just the study's standard N=21): inference cost rises to 2.29 s/sample, about 500× the published 4.6 ms/sample number -- a real, sublinear (~n^0.74) but far from flat scaling with mesh size. Still 59× faster than torch-fem's own matched-precision solve at that size, but nowhere near the thousands-of-times gap reported at the standard resolution. The report's “inference cost is essentially flat in mesh size” line (true within its own tested range, up to N=49) now has this caveat added directly in both documents. Accuracy at N=1401 has since been checked directly against a real finite-element ground truth (round-10 follow-up below), which also uncovered and fixed a checkpoint-loading bug and used a subsequent multi-resolution retraining to close most of the resulting accuracy gap.

Figure 41. Neural-operator inference time vs. torch-fem solve time at N=1401: 2.29s vs. 133.83s (matched precision) and 29.11s (old, unmatched) -- still fastest, by a much smaller margin than at standard resolution.

Point 3 -- batch-size table removed.

Removed Table 6 (validation error vs. batch size) and its three figures from both documents, per your note that it was confusing. Fixed the numbering/ordering throughout both documents so nothing is left with a gap or a dangling reference to it.

Point 4 -- torch-fem solver description, fixed.

Found and fixed a real inconsistency you caught: one paragraph described torch-fem's iterative preconditioner setup, and the very next one described it as a direct factorization -- they contradicted each other and didn't match what this comparison actually ran (CG, Jacobi-preconditioned, never a direct factorization in this study). Fixed to be consistent.

Point 5 -- TensorMesh, working and verified.

Confirmed the library itself has what you described (Q4 and Q9 elements, a Newton solver, a direct GPU solver via cuDSS), and got a small Newton + direct-solver case running with Q4 -- Q9 has not been attempted yet, so this result speaks to Q4 only. Cross-checking against our own solver first showed a real disagreement, initially traced to what looked like a bug in TensorMesh's own quadrature for quad elements. Re-investigating rather than accepting that verdict found the actual cause: TensorMesh's quadrilateral element expects node ordering in tensor-product order (bottom-left, bottom-right, top-left, top-right), not the perimeter order this project's own mesh generator (and meshio/VTK/torch-fem) uses -- confirmed directly from TensorMesh's own official mesh generator output, not assumed. With that one-line node-order fix, a real Newton + direct-solver (LU) run on B1 x Neo-Hookean now matches our own solver to about 10 significant digits. TensorMesh is confirmed correct and usable for this comparison. Measured directly how far its default (dense) Jacobian scales, rather than assuming: timed real solves at N=3/11/21/31/51, giving 0.13s/0.21s/2.64s/14.53s/105.27s -- an empirical ~dof^2.2 growth, projecting roughly 10 days for a single solve at torch-fem's own smallest production resolution (N=401). Per your instruction to fix this rather than stop at N=51, wrote an explicit sparse Jacobian (build_sparse_jac_fn), reusing this project's own already-correct matrix-free Hessian machinery instead of the library's dense default. Real result: N=51 dropped from 105.27s to 0.42-0.49s (~215-250x faster), with IDENTICAL accuracy (1.212e-11 relative difference vs. our own solver, unchanged). Then ran the full production sweep on GPU (N=401/701/1001/1401, matching torch-fem's own resolutions exactly, real cuDSS direct solver forced past the library's own conservative 2M-DOF auto-fallback threshold): TensorMesh's L2 relative error matches torch-fem's own to every printed digit at all four resolutions (2.500e-05 / 1.009e-05 / 5.155e-06 / 2.291e-06), and TensorMesh is consistently FASTER than torch-fem at every one of them -- 1.92x/1.77x/1.80x/2.13x, including at N=1401 (3.9M DOF), with no degradation crossing the library's own 2M-DOF threshold. This closes the comparison completely: TensorMesh, run correctly (Newton + a genuine direct solver, not L-BFGS), is both accurate and faster than torch-fem across the full production range you asked about.

Figure 45. TensorMesh (sparse jac_fn, real cuDSS direct solver) vs. torch-fem at production scale (N=401-1401): L2 relative error identical to every printed digit; TensorMesh 1.8-2.1x faster at every resolution.

Point 6 -- timing breakdown by phase (assembly/solve/iterations/peak memory).

Measured directly for torch-fem (N=401-1401), all four quantities you asked for -- total time, assembly, solve/factorization, nonlinear iterations, and peak memory: assembly is 43% of total time at N=401, falling to 24% at N=1401 as the CG solve itself scales worse than assembly with problem size; the nonlinear (Newton) iteration count is a constant 11 at every resolution tested and for both solvers (CG and direct), so the growth in solve time with N is entirely a per-iteration cost effect, not more iterations being needed at larger N. Also tried a direct (LU) solve, per your suggestion -- real, unexpected result: it is 14x SLOWER than CG+Jacobi at N=401 and 33x slower at N=701, growing worse with N, not better (not attempted above N=701 given that trend). "Ours" has no separate assembly phase to report by architecture -- every CG iteration is itself a Hessian-vector product via automatic differentiation, not a distinct matrix build -- its own iteration counts and peak memory were already reported.

On your main concern directly -- whether our GPU FEM is a competitive baseline at all, not just slower: torch-fem is genuinely faster at every size both solvers can run, and that is not in question. But torch-fem's own peak GPU memory at these same resolutions (5.7/17.3/35.3/69.2 GB at N=401-1401) is already within about 13% of an 80GB A100's own ceiling at the largest size tested, while our solver ran that same N=1401 case on the same hardware without a comparable memory wall, because it never assembles a global matrix at all. Combined with the direct-solver finding above (a real attempt to make torch-fem's own solve faster, which made it slower instead), the evidence points to two solvers built for different questions -- how fast at a size that fits, vs. how far this can go on one GPU -- rather than one being an under-optimized version of the other. Happy to restructure the report so torch-fem is the primary GPU-FEM baseline at reachable sizes, with our matrix-free solver's role reserved for sizes beyond that, if that is a more useful framing for the paper.

Figure 42. torch-fem timing breakdown by phase: assembly vs. CG-solve time, CG vs. direct(LU) solve, and peak GPU memory, N=401-1401.

Point 7 -- tolerance sensitivity (1e-6/1e-7 vs. 1e-8), optional.

Per your own suggestion ("if you wish"), re-ran torch-fem at N=401 and N=1401 (the smallest and largest resolutions in the matched-precision sweep) at tol=1e-6 and 1e-7, alongside the already-used 1e-8. Result: the L2 relative error is identical to 4 significant digits across all three tolerances at both resolutions (N=401: 2.4996e-05 at 1e-6/1e-7 vs. 2.5000e-05 at 1e-8; N=1401: 2.2911e-06 at 1e-6/1e-7 vs. 2.2910e-06 at 1e-8) -- loosening the tolerance buys no measurable accuracy loss but also no measurable accuracy gain either way. Wall-clock differs by only 2-10% between the tightest and loosest tolerance tested (1.02x at N=401, 1.10x at N=1401), smaller than typical run-to-run noise on a shared GPU. Conclusion: 1e-8 costs essentially nothing extra here, so the report keeps it as the standard tolerance throughout rather than trading a real, verifiable precision guarantee for a marginal and inconsistent speed gain.

Figure 43. torch-fem tolerance sensitivity: L2 relative error and wall-clock time at tol=1e-6/1e-7/1e-8, N=401 and N=1401.

Point 8 -- experimental, open question: an assembled+direct variant of our own solver.

Since torch-fem and TensorMesh’s own explicit-assembly + direct-solve approach was already working correctly and fast, without ever exhausting memory, at every resolution tested (N=401-1401, at most ~69GB of an 80GB A100), we tried the same approach for our own solver instead of assuming it must stay matrix-free unconditionally: reusing our own already-verified per-element Hessian machinery to assemble an explicit sparse tangent, then factorizing it directly with the same GPU direct solver (cuDSS) TensorMesh uses -- no TensorMesh dependency involved, this is our own physics, just assembled instead of matrix-free. Verified correct against our own matrix-free solver’s own converged result (N=11: 1.2e-11, N=21: 1.24e-11 relative difference) before trusting any timing, the same discipline used throughout this study.

Real result on the same A100, N=401-2001: accuracy matches torch-fem and TensorMesh to every printed digit at every resolution, and peak memory is roughly 3.5-4.5x lower than torch-fem’s at every size – comfortably inside the memory limit with far more headroom than either existing baseline. A memory-per-DOF line fitted from the first four resolutions predicted peak memory at two new, larger resolutions (N=1701, N=2001) to within 0.15% – a real out-of-sample check, not curve-fitting after the fact – and projects our own ceiling on this GPU near N≈3200 DOF-wise, versus torch-fem’s own line already close to its limit near N≈1500.

| N | ours (assembled+direct) | torch-fem (cg) | TensorMesh | ours peak mem | torch-fem peak mem |
| --- | --- | --- | --- | --- | --- |
| 401 | 4.75 s | 9.82 s | 5.11 s | 1.3 GB | 5.8 GB |
| 701 | 12.99 s | 25.15 s | 14.17 s | 3.9 GB | 17.7 GB |
| 1001 | 28.42 s | 56.65 s | 31.43 s | 8.0 GB | 36.1 GB |
| 1401 | 58.54 s | 133.83 s | 62.96 s | 15.6 GB | 70.8 GB |
| 1701 | 91.53 s | — | — | 23.1 GB | — |
| 2001 | 127.44 s | — | — | 31.9 GB | — |

The torch-fem numbers above are its own cg (iterative) option, not a direct solve. Architecturally matched (direct vs. direct), the gap is much larger – torch-fem’s own direct solve was so much slower than its cg solve that it was never even attempted past N=701:

| N | ours (direct) | torch-fem (direct, real) | speedup |
| --- | --- | --- | --- |
| 401 | 4.75 s | 166.57 s | 35× |
| 701 | 12.99 s | 835.58 s | 64× |

A further optimization on top of this – reusing the direct solver’s own one-time matrix reordering computation across every Newton iteration within one solve, instead of redoing it from scratch each iteration (found by reading the underlying library’s own source, then confirmed for real: that reordering step was 95.7% of one linear solve’s own cost) – gave a further, independently verified 2.0-2.4x END-TO-END speedup at the same resolutions, with identical accuracy, at the cost of a modest ~30-40% increase in peak memory (still roughly 3.5x lower than torch-fem’s).

This is a genuinely different solver variant from the matrix-free one reported everywhere else in this study, built and verified this week, and not yet finalized on our side. We would like your read on it before treating it as more than a promising experiment: does this change how you’d want the GPU-FEM comparison framed, and should this become the primary comparison alongside – or instead of – the matrix-free result, given it now appears faster and lower-memory than both established baselines at every size tested so far? We are continuing to look for ways to make it faster and more memory-efficient still before drawing a final conclusion.

Point 9 -- further optimization on the assembled+direct variant, with full engineering detail for your review.

Continuing to push on the experimental solver from Point 8, we found and verified two further, purely engineering optimizations to its direct-solve step -- documenting the full detail here, not just a final number, so you can judge on the same footing we have whether this was done correctly and whether it is worth pursuing.

(1) Reusing the direct solver’s own matrix reordering. The underlying library (torch_sla, via NVIDIA’s cuDSS) recomputes a full matrix-reordering ("analysis") step from scratch on EVERY Newton iteration, even though that step depends only on the matrix’s sparsity pattern, which never changes within one Newton solve – only the numeric values do. Measured directly, before writing any new code: this reordering step was 95.7% of one linear solve’s own cost at N=401. We wrote a custom Newton loop that computes it once per solve and reuses it every iteration thereafter. Verified bit-for-bit identical to the original solver at N=11 (relative difference 3.5e-16) before trusting any timing.

(2) Symmetric matrix storage. The assembled tangent matrix is mathematically symmetric (it is the Hessian of a scalar energy), but was being stored and factorized as a general (non-symmetric) matrix, because of how fixed boundary-condition rows were eliminated during assembly. Since our own Dirichlet boundary conditions hold the fixed displacements at exactly zero throughout every Newton iteration, we confirmed a small change to that elimination makes the stored matrix genuinely symmetric with ZERO change to the physical solution – verified bit-for-bit identical (0.0 relative difference, not merely close) through a complete nonlinear solve at N=11 and N=21, entirely on CPU, before this was ever run on a GPU. This lets the direct solver use a cheaper symmetric factorization instead of a general one.

Real, GPU-verified progression at N=1401 (wall-clock, peak memory):

| Configuration | N=1401 wall-clock | N=1401 peak memory |
| --- | --- | --- |
| Assembled+direct only (Point 8) | 58.54 s | 15.6 GB |
| + analysis-reuse | 24.61 s | 20.5 GB |
| + analysis-reuse + symmetric storage | 23.68 s | 17.1 GB |

Accuracy is identical to Point 8’s own numbers at every resolution across all three rows above.

As with Point 8, this is not a finalized result on our side. We would like your read on whether the underlying approach, and the depth of optimization now built on top of it, is sound engineering worth continuing, and how you would like it treated relative to the matrix-free solver reported everywhere else in this study – we can keep looking for further speed, but wanted your judgment on this before investing more time in the same direction.

## Response to Timon's round-10 feedback (accuracy-matched comparison, throughput, profiling, geometry, break-even)

New since the round-9 reply already sent: (1) a checkpoint-loading bug found and fixed, which had been silently corrupting every round-10 accuracy result with a completely different (data-driven baseline) model; (2) the operator's own accuracy at N=1401 checked directly against real FEM ground truth for the first time, and the coarsest FEM mesh that matches it; (3) a multi-resolution retraining that reduced the error at N=1401 from 44.65% to 5.85% and inverted the peak-stress crossover finding; (4) batch-size/throughput and profiling/torch.compile/TF32 re-verified against both the corrected and the retrained checkpoint, confirming both are checkpoint-independent; (5) a real accuracy-matched break-even. One question remains open and blocks further work: point 4's complex-geometry design (ring with a local notch), already verified with real mesh-convergence evidence, is awaiting Timon's confirmation before any training time is committed to it.

Point 1 -- accuracy-matched comparison (operator vs. coarsest suitable FEM), not just matched-resolution.

Answering this required first finding and fixing a real bug: every round-10 script loading the B1 x Neo-Hookean checkpoint tried a storage path that never existed and silently fell back to a completely different (data-driven baseline) model -- caught via a flat, resolution-independent ~620-640% error at every N, the signature of the wrong model rather than a real accuracy failure. Fixed by resolving the checkpoint by content (SHA-256 fingerprint) instead of a hardcoded path.

With the correct checkpoint: operator accuracy at N=1401, checked directly against real FEM ground truth for the first time, is 44.65% relative L2 (39.49% L2-norm, 61.16% H1 semi-norm), degrading smoothly away from the training resolutions (best ~7.4% near N=33-37). Against native FEM at low N (3-49): a near-degenerate mesh already matches or beats the operator's accuracy in L2, H1, energy, and reactions. Peak PK1 stress needs a caveat -- evaluated at a fixed location from a fine reference (fixing an earlier metric-definition mismatch), both methods converge to it very slowly, and the located point sits essentially at a domain-corner boundary-condition transition, a classic near-singularity location in elasticity.

Given the degradation, retrained a new checkpoint on a wider resolution spread (N=21,33,101,201 instead of 21,33 only), using a GPU-accelerated data path verified bit-identical to the original solver first. Result: the monotonic degradation is gone. Error drops from 44.65% to 5.85% at N=1401 (7.6x), and instead of climbing indefinitely, plateaus around 5.8-5.9% beyond N~50.

| N | Original checkpoint (N=21,33 only) | Retrained (N=21,33,101,201) |
| --- | --- | --- |
| 13 | 13.85% | 11.74% |
| 33 | 7.37% | 3.50% |
| 49 | 8.50% | 2.32% |
| 101 | 14.99% | 3.55% |
| 201 | 22.28% | 4.85% |
| 701 | 34.34% | 5.88% |
| 1401 | 44.65% | 5.85% |

Table R10-1. disp_rel_L2 vs. real FEM ground truth, original vs. retrained checkpoint.

Figure R10-A. Original checkpoint's accuracy degradation, N=49 to N=1401 (pre-retraining).

The retrained checkpoint also flips the peak-stress finding: its peak-stress accuracy (42-70%, improved at every N) is now BETTER than anything FEM achieves in the whole low-N range (62% at best) for every operator resolution N>=29 -- FEM would need N>49 (untested) to match it. The "coarsest suitable FEM" is now bound by tangent energy, not peak stress, and dropped from N=17-45 (original checkpoint) to a near-degenerate N=9-17 (retrained checkpoint) for almost every resolution. Same domain-corner caveat still applies to peak stress specifically -- the improvement itself is real and independently verified, not a training-validation artifact.

The same fix (retrain on N=21,33,101,201 instead of N=21,33 only) was since extended to every other case with the same degradation signature. Protocol, exactly as the advisor asked: weighting is equal by construction (400 train / 100 val samples per resolution, identical count at all four); each batch is homogeneous in one resolution, but the full batch list spanning all four is shuffled together so different resolutions interleave randomly through the epoch; the loss carries no explicit cross-resolution weight. N=101/201 were added because they were exactly where the degradation sweep had already measured real error (15.0%/22.3%) before any retraining ran. Two methods changes made the later retrains cheaper: TF32 training (verified safe, 2.08x at N=201, the resolution it actually helps), and a real bug found and fixed this session -- the GPU fast data-generation path had only ever been wired up for B1, so B2's own --fast_solver/--nsteps did nothing at all until now; every earlier B2 job silently ran on the slow CPU solver regardless of its own launch command.

Results, N=1401, old vs. new checkpoint (Table R10-2): B1 x Mooney-Rivlin 39.20% -> 15.04% (~62% relative reduction); B1 x Arruda-Boyce 45.62% -> 25.05% (~45%, and its own real GPU run is reassuring evidence that a shared chain-locking-clamp exposure with this project's own torch-fem Arruda-Boyce runs does not bite in practice here -- all 16 resolutions, both checkpoints, converged cleanly); B2 x Mooney-Rivlin 49.47% -> 13.10% (~73.5%, the largest reduction so far, and the clearest case of a genuinely FLAT error across N=37-1401 rather than merely a smaller one at N=1401); B2 x Neo-Hookean 46.37% -> 36.05% (~22.3%, the smallest of the five reductions -- unlike Mooney-Rivlin's flat curve, this checkpoint's error is lowest right where it was trained, N=101 at 12.98%, and rises again beyond that to 36-40% by N=1401, still clearly better than the original checkpoint's 46.4-46.7% there).

Round-11 point 4 -- direct-N1401 training ablation -- has since completed. A B1 x Neo-Hookean checkpoint trained DIRECTLY at N=1401 (100 samples, early stopped at epoch 36, best epoch 20) is cheaper to train (8.00h vs. 11.63h, ~30% less GPU time) but 6.3x LESS accurate (36.65% vs. 5.85% disp_rel_L2) than the multi-resolution zero-shot checkpoint, with essentially identical inference cost (2318.8 ms vs. 2292.1 ms/sample). A genuine result in favour of the multi-resolution strategy: not just "it also works zero-shot," but "it produces a substantially better model for less than 50% more training time." Table R10-5.

| Metric | Direct @N=1401 | Multi-res (zero-shot) |
| --- | --- | --- |
| Training wall-clock | 8.00 h | 11.63 h |
| disp_rel_L2 @ N=1401 | 36.65% | 5.85% |
| Inference (eager fp32) | 2318.8 ms | 2292.1 ms |

Table R10-5. Direct-N1401 training ablation vs. multi-resolution zero-shot, B1 x Neo-Hookean.

Figure R10-E. Direct-N1401 training vs. multi-resolution zero-shot, training cost and accuracy (Table R10-5).

| Case | Old (N=1401) | New (N=1401) | Relative reduction |
| --- | --- | --- | --- |
| B1 x Neo-Hookean | 44.65% | 5.85% | ~87% (7.6x) |
| B1 x Mooney-Rivlin | 39.20% | 15.04% | ~62% |
| B1 x Arruda-Boyce | 45.62% | 25.05% | ~45% |
| B2 x Mooney-Rivlin | 49.47% | 13.10% | ~73.5% |
| B2 x Neo-Hookean | 46.37% | 36.05% | ~22.3% |

Table R10-2. Multi-resolution retrain fix, every completed case, disp_rel_L2 at N=1401.

Figure R10-B. Multi-resolution retrain fix, old vs. new checkpoint, all completed cases (Table R10-2).

Point 2 -- batch size and throughput at matched GPU memory.

| Scenario | Max batch size | Peak GPU memory | Throughput |
| --- | --- | --- | --- |
| Operator (own ceiling) | 8,192 | 47.6 GB | 4,203.79 samples/s |
| FEM (own ceiling) | 256 | 38.7 GB | 2.86 samples/s |
| FEM (capped at operator's 47.6 GB) | 256 | 38.7 GB | 2.86 samples/s |
| Operator (capped at FEM's 38.7 GB) | 4,096 | 23.8 GB | 4,207.75 samples/s |

Table R10-2. Max feasible batch size and throughput, N=21.

Figure R10-B. Throughput and peak memory vs. batch size, operator vs. FEM, own memory ceilings.

Peak throughput favors the operator ~1,470x regardless of memory cap -- FEM's own ceiling is already below saturation, while the operator barely gives up anything capped down to FEM's much smaller budget. Batching helps the operator ~20x (bs=1 to its own ceiling) while FEM barely benefits (~5x), matching Timon's own stated expectation. Re-measured against both the corrected and the retrained checkpoint: max-batch-size throughput 4,182.27 vs. 4,203.79 samples/s (0.5% relative difference) -- checkpoint-independent, as expected for a timing measurement.

Point 3 -- NO inference optimization and profiling.

Pure GPU forward-pass time, fp32, batch size 1, N=1401: 2,290 ms/sample. Peak memory 23.16 GB, higher than the GPU-FEM solver's own peak at the same N (15.6-20.5 GB). Profiler: ~67% in batched matmuls, ~22% in MLP layers -- genuine core compute, not an obvious inefficiency.

| Variant | ms/sample | Speedup vs. eager | Output diff vs. eager |
| --- | --- | --- | --- |
| Eager (fp32) | 2,292.1 | 1.00x | -- |
| torch.compile | 2,145.1 | 1.07x | 6.0e-7 |
| Eager + TF32 | 491.2 | 4.67x | 3.2e-3 |
| Compile + TF32 | 394.0 | 5.82x | 3.2e-3 |

Table R10-3. Inference-speed optimization attempts, N=1401, correctness-checked against strict fp32 eager.

Figure R10-C. Same data as Table R10-3.

torch.compile alone: 1.07x (correctness-checked). TF32 matmul precision: 4.67x alone, 5.82x combined with torch.compile -- the best result found, at a real but small accuracy cost (~3.2e-3 relative), checked directly. Re-measured against both the corrected and the retrained checkpoint: all four variants within 0.01-0.8% of the original numbers (eager 2,286.9 vs. 2,292.1 ms; compile 2,145.2 vs. 2,145.1 ms; eager+TF32 487.3 vs. 491.2 ms; compile+TF32 395.0 vs. 394.0 ms) -- checkpoint-independent as expected.

Point 4 -- complex-geometry example, one question before committing training time.

Designed and verified a candidate before spending GPU-hours training: extending the pressure-vessel-like ring case with a single smooth local notch (Gaussian dimple) in the inner wall -- still simply connected, reusing existing solver machinery. Mesh-convergence check across four resolutions confirms a genuine local stress concentration: peak stress at the notch still rising (11.70 -> 13.94, +3.7% at the finest mesh, still rising) while global displacement is essentially flat (+0.27%) -- exactly the signature Timon described of a globally-converged field hiding an under-resolved local quantity. Awaiting his confirmation of this design before training a full model on it.

Point 5 -- break-even analysis, accuracy-matched.

Using the retrained checkpoint's own coarsest-suitable FEM at N=1401 (N=11, bound by tangent energy per the crossover above) against the operator's real inference cost there, and the real training wall-clock for the retrained checkpoint (41,881 s, ~11.6h -- confirmed from its own training log; it stopped itself via early stopping, 8 checks with no improvement, exactly as configured):

| NO inference mode | ms/sample @ N=1401 | vs. FEM@N=11 (1,625.6 ms) | Break-even |
| --- | --- | --- | --- |
| Eager fp32 (default) | 2,292.1 | 0.71x (slower) | Never -- accuracy-matched FEM already cheaper |
| Compile + TF32 | 394.0 | 4.13x (faster) | After 34,005 samples (~3.72 GPU-hours) |

Table R10-4. Accuracy-matched break-even: operator@N=1401 vs. FEM@N=11. Distinct from the earlier matched-resolution break-even (Point 3 above): this one intentionally runs the two methods at different N, matched by accuracy rather than by mesh.

This is the one place in the whole round-10 investigation where the operator's default (unoptimized) mode does not come out ahead: run naively, it never repays its own training cost against a FEM mesh that is only as accurate as it needs to be. With the point-3 optimizations (torch.compile + TF32), it both wins per-sample (4.13x) and repays its training cost cheaply -- about 3.72 GPU-hours against 11.6 GPU-hours of training. The operator's economic case depends on deploying it with these optimizations, not on its default settings -- reported honestly rather than only the favorable case.

The advisor's round-11 feedback asked to keep a SECOND break-even comparison too: both methods at the SAME resolution, N=1401, rather than each at its own accuracy-matched mesh -- expected, correctly, to look much more favourable to the operator. Table R10-4', all six cases, operator's own default eager fp32 forward pass at N=1401 vs. torch-fem's own real GPU solve time at the same N.

| Case | torch-fem @N=1401 | Operator @N=1401 | Speedup | Break-even |
| --- | --- | --- | --- | --- |
| B1 x Neo-Hookean | 135.00 s | 2292.1 ms | 58.9x | 316 samples |
| B1 x Mooney-Rivlin | 133.92 s | 2361.9 ms | 56.7x | training cost unknown |
| B1 x Arruda-Boyce | FAILED (see note) | - | - | - |
| B2 x Neo-Hookean | 205.14 s | 2351.1 ms | 87.3x | training cost unknown |
| B2 x Mooney-Rivlin | 207.21 s | 2356.0 ms | 88.0x | training cost unknown |
| B2 x Arruda-Boyce | FAILED (see note) | - | - | - |

Table R10-4'. Resolution-matched break-even, both methods at N=1401, all six cases. At matched resolution the operator wins by 57-89x even unoptimized -- unlike Table R10-4's own accuracy-matched comparison, where the default mode does not break even at all. All four non-Arruda-Boyce numbers are from a single clean re-run (2026-09-17) after fixing a real memory-cleanup bug in the sweep script (an exception-chain reference kept a failed case's own GPU tensors pinned into the next case) -- this re-run succeeded for every non-Arruda-Boyce case in one pass, confirming the fix on real GPU. Both Arruda-Boyce cases fail genuinely and consistently across every run: torch-fem's own Newton-Raphson solve does not converge at N=1401, root-caused to a real CUDA OOM inside torch-fem's own Hessian assembly for this specific material.

Figure R10-D. Resolution-matched speedup, all six cases (Table R10-4'). (Figure R10-C already exists for Table R10-3's own data.)

Task #22's peak-stress QoI, the five cases beyond B1xNeo-Hookean (Table R10-6). Caveat: B1xMooney-Rivlin, B1xArruda-Boyce and B2xMooney-Rivlin here reflect their ORIGINAL pre-retrain checkpoints (their disp_rel_L2 matches the "OLD" column in Table R10-2 exactly) -- the peak-stress metric has not been recomputed against this week's new retrained checkpoints for those three. B2xNeo-Hookean and B2xArruda-Boyce have only one checkpoint each, no caveat.

| Case | disp_rel_L2 @N1401 | peak_stress_rel_err @N1401 |
| --- | --- | --- |
| B1 x Mooney-Rivlin | 39.20% | 78.05% (orig. checkpoint) |
| B1 x Arruda-Boyce | 45.62% | 78.25% (orig. checkpoint) |
| B2 x Neo-Hookean | 46.37% | 49.48% |
| B2 x Mooney-Rivlin | 49.47% | 48.13% (orig. checkpoint) |
| B2 x Arruda-Boyce | 39.73% | 39.97% |

Table R10-6. Peak-stress QoI at N=1401, five cases. B1's own two rows (78.05%, 78.25%) are structurally worse than every B2 case (39.97%-49.48%), consistent with B1's own boundary-corner stress-singularity caveat noted above. B2xArruda-Boyce (39.97%) has the BEST peak-stress accuracy of all six cases despite failing the break-even above -- unrelated findings: that failure is torch-fem's own reference solve running out of memory, not a property of the operator's own accuracy.

Figure R10-F. Peak-stress QoI vs. displacement error, five cases (Table R10-6).

## Summary of what was done and what came out

Reference finite-element solution.

I systematically checked mesh convergence for all six geometry–material combinations, using meshes from N=6 to N=51. At the mesh resolution used throughout the study (N=21), both monitored quantities change by less than 0.2% between successive refinements, and by N=51 the change falls below 0.02%. The FEM reference solutions used throughout the report can therefore be considered well converged.

High-DOF reference and element-order comparison (B1 × Neo-Hookean).

I computed high-resolution reference solutions at N=2236 using both Q4 and Q9 elements, corresponding to 9,999,392 and 39,979,682 DOF, respectively, and evaluated the convergence rates against these references. As expected from finite-element theory, Q9 converges faster than Q4 in all three norms. The observed convergence rates are 1.57, 0.76, and 0.75 for Q9 in the L2, H1, and tangent-energy norms, respectively, compared with 1.39, 0.72, and 0.71 for Q4. A direct comparison between the two fine reference solutions gives a relative L2 difference of 2.06×10⁻⁶, which satisfies the 10⁻⁵ target. However, the H1 semi-norm difference (≈1.15×10⁻³) and tangent-energy norm difference (≈9.61×10⁻⁴) remain above that threshold. The most likely explanation is that the Q4 reference still contains noticeable discretization error relative to Q9, even at the same N. The Q9 fine solve also reached the allocated CG iteration limit during 6 of the 20 Newton iterations, so the exact norm values should be interpreted with a small additional numerical margin. Newton convergence itself was achieved, however, and the overall conclusion remains unchanged.

Computational cost.

The previous fixed estimate of 8.0 s/sample has been removed. FEM cost is now measured directly for each individual case and ranges from 25.4 to 61.7 s/sample. The reported timings are obtained in FP64 and are separated into assembly and solver components, with the corresponding solver settings and FLOP counts also documented. In addition, I implemented a GPU-native FEM solver and verified it against the CPU reference to machine precision. When sufficiently batched, the GPU implementation is 71.7–171.5× faster than the corresponding CPU FEM solve. Once the neural operator has been trained, inference requires only 4.6–5.0 ms per prediction, corresponding to a speedup of approximately 5,387× to 12,575× relative to solving the same problem using FEM.

Accuracy–cost trade-off and break-even.

I placed the trained operator and the finite-element solver on a single accuracy-versus-cost comparison for all six geometry–material combinations, each scored against a common N=101 reference across nine mesh resolutions (N=13 to 49). The finite-element solver is more accurate at every resolution tested; the operator's case rests entirely on cost, since its inference time is nearly flat with mesh size while FEM cost grows with N, within the small resolutions actually tested here (N=13 to 49). Measured directly at N=1401 later (2026-09-10, far outside this range), inference cost is NOT flat there -- it rises to 2.29 s/sample, a real but sublinear (~n^0.74) growth with mesh size, though still faster than a matched-precision FEM solve at that same size. Across all six cases the resulting speed-up ranges from 1,630× to 60,651×. For the two geometries' checkpoints jointly trained at two resolutions (N=21 and 33), the operator's error shows a clear local minimum exactly at those two resolutions most pronounced for B2 × Neo-Hookean (4.30× and 3.65× more accurate there than at neighboring resolutions) and present, more weakly, in B2 × Mooney-Rivlin (3.07×/3.17×) and B2 × Arruda-Boyce (2.99×/2.47×) as well. Using these matched timings, the break-even point the number of new problem instances needed before the operator's one-off training cost is repaid is 52–1,245 new samples against the CPU reference solver, and 1,133–95,038 against the GPU-native solver, depending on the case and the batch size at which the GPU solver is run.

Identical batch-size benchmark.

I benchmarked the trained operator against the GPU-native FEM solver at the same batch sizes (1, 8, 32 and 128) for every case. At batch size 1 the operator is 356×–544× faster than the GPU FEM solver; at batch size 128, where FEM benefits most from batching, the advantage narrows but remains 1,215×–1,297×.

Batch size.

The previous batch-size comparison was not fully fair because every batch size was trained for the same number of epochs. Consequently, smaller batches received substantially more optimizer updates than larger batches. I therefore repeated the comparison using an equal training budget of 40,000 optimizer steps for every batch size. With a fixed learning rate, the resulting accuracy is essentially flat across batch sizes from 8 to 256, with errors ranging from 0.098 to 0.115. This shows that the degradation observed previously was primarily caused by the unequal number of optimizer updates rather than by batch size itself.

B2 accuracy.

The B2 cases previously showed much larger errors than B1, at approximately 31–32%. I traced this discrepancy to the boundary-force representation used during training. The training objective used the raw pressure value rather than the FEM-consistent, edge-weighted nodal force, which is approximately 13–16× larger along the curved boundary. Replacing the raw pressure with the correct nodal force alone actually increased the error to approximately 94%, because this force term is also the contribution that drives the network away from the trivial zero solution. I therefore normalized the training objective using each sample's own force scale, which preserves the true minimizer while balancing the loss terms. With this correction, the three B2 material cases now achieve errors of 9.11%, 7.28%, and 9.81%, respectively, bringing them into the same accuracy range as B1. This confirms that B2 was not intrinsically more difficult; the previous degradation resulted from a defect in the training objective.

Error in physically important quantities beyond displacement.

On the same six trained checkpoints, evaluated over 50 held-out samples, I measured the error in the H1 semi-norm, the tangent-elastic-energy norm, the first Piola–Kirchhoff stress tensor (aggregate, per component, and its maximum over the mesh), and the reaction force on the constrained boundary. All of these exceed the displacement error of the same case (7.21–11.71%): H1 semi-norm 10.47–24.20%, tangent energy 11.37–17.02%, aggregate stress 13.32–18.14%, reaction-force resultant 4.66–12.61%. Peak stress is the sharpest outlier and splits the two benchmark geometries: 5.38–5.99% on B2 against 19.87–47.53% on B1.

Out-of-distribution behaviour.

Each trained model was evaluated without retraining material parameters and loading levels outside the range encountered during training, specifically using stiffer materials and larger loads. All six geometry–material combinations show a similar degradation factor of approximately 3.9–5.6×. This indicates that the models learn the training distribution effectively but do not yet extrapolate the underlying physical behaviour robustly outside that distribution.

Resolution invariance.

Following your clarification, the resolution-invariance experiment uses a single model, trained once on two resolutions (N=21 and 33), and evaluated without any retraining on seven mesh resolutions not seen during training, all compared against the same fine N=101 reference solution. This has now been completed for all six geometry–material combinations. All three B1 materials stay between 5.0% and 10.6% error across every unseen resolution; the three B2 materials show the same property, real but weaker than B1's. The direct computational saving this demonstrates is that one trained checkpoint serves seven additional resolutions with no further training, avoiding a separate training run which costs between 2,785 and 34,164 seconds per case for each of those seven resolutions.

GPU-native FEM at finer discretization’s.

I extended the GPU-native solver benchmark, validated earlier to machine precision against the CPU reference, up to N=1401 (3,925,602 degrees of freedom, close to four million), where it completes in 11.0 hours. The solver was written from scratch rather than built on Tensor Mesh. I checked Tensor Mesh directly and its element library is triangular/tetrahedral/pyramid/prismatic only, with no quadrilateral (Q4/Q9) elements, which this entire study is built on, so it could not have been used for a like-for-like comparison here. torch-fem is a closer match PyTorch-native, GPU-accelerated, with Q4/Q9-equivalent quadrilateral elements. You confirmed (Sep 6) you have no preference between TensorMesh and torch-fem, since the only requirement is an efficient GPU implementation for a fair comparison to a neural operator torch-fem satisfies this, so it was used for the direct comparison in item #13 below.

Manufactured-solution comparison (MMS).

Rather than relying on FEM as the reference, I constructed a manufactured solution with a known closed-form displacement field and the corresponding body force, giving an exact ground truth to score both FEM and the trained operator against, on the same footing. At N=17, the standard Q4 discretization reaches a relative L2 error of 3.403×10⁻³ and Q9 reaches 5.163×10⁻⁵, both converging close to their theoretical rates as the mesh is refined. The trained operator does not converge under the same refinement: its error is 8.238×10⁻³ at N=17 and gets worse, not better, as the mesh is refined further (a fitted rate of −0.59), because its error is dominated by its own optimization error rather than by mesh discretization error. This held up when repeated over the operator's own 16-member test family, not only the single problem instance used above.

GPU-native solver vs. torch-fem (item #13).

I compared the matrix-free GPU-native solver directly against torch-fem, an established, GPU-accelerated finite-element library, on B1 x Neo-Hookean at N=401/701/1001/1401. torch-fem wins wall-clock by a large, non-monotonic margin (418x/655x/1197x/936x, peaking at N=1001) -- reported honestly with two caveats: the two solvers are not run at matched precision/tolerance (torch-fem float32/loose vs. ours float64/tight), and our own peak GPU memory was not captured (resumed from a checkpoint by design). This matches Timon's own round-8 prediction that an assembled-matrix solver would beat a matrix-free one on raw speed.

Figures for every result table (item #12).

Every table in this document and the full Report that lacked a figure now has one -- 39 figures total, covering mesh convergence, training cost, latency and speed-up, break-even, in- vs. out-of-distribution error, the B2 accuracy fix history, physical-quantity errors, operator-vs-FEM accuracy and cost, solver scaling, the torch-fem comparison, MMS convergence rates, and physics-informed vs. data-driven training, plus field-grid visualizations (mesh convergence, zero-shot resolution, out-of-distribution shift, and DD-NO coarse-vs-fine) generated by adding a small opt-in plotting flag directly to the original, already-validated study scripts -- never a separate reimplementation. One real rendering bug was found and fixed along the way: the B2 ring geometry's own inner hole was being incorrectly filled in by a generic point-cloud triangulation; fixed by triangulating from the real mesh connectivity instead.

## 1. Mesh refinement / spatial convergence

Confirmed for all 6 cases (h-refinement sweep, N=6–51). B1’s Q4-vs-Q9 check against a ~10M/40M-DOF reference is also done. B2’s DOF-referenced study is not started yet.

| N | Nodes | Max ¦u¦ | Δ vs. prev. | Strain energy U | Δ vs. prev. |
| --- | --- | --- | --- | --- | --- |
| 6 | 36 | 5.8693e-3 |  | 1.7599e-2 |  |
| 11 | 121 | 5.9484e-3 | 1.347% | 1.7673e-2 | 0.420% |
| 16 | 256 | 5.9518e-3 | 0.057% | 1.7692e-2 | 0.108% |
| 21 (study mesh) | 441 | 5.9606e-3 | 0.149% | 1.7700e-2 | 0.046% |
| 26 | 676 | 5.9606e-3 | 0.001% | 1.7705e-2 | 0.025% |
| 31 | 961 | 5.9636e-3 | 0.051% | 1.7707e-2 | 0.015% |
| 41 | 1681 | 5.9648e-3 | 0.021% | 1.7710e-2 | 0.017% |
| 51 | 2601 | 5.9655e-3 | 0.011% | 1.7712e-2 | 0.009% |

Table 1. B1 x Neo-Hookean mesh convergence (N=6-51)

| N | Nodes | Max ¦u¦ | Δ vs. prev. | Strain energy U | Δ vs. prev. |
| --- | --- | --- | --- | --- | --- |
| 6 | 36 | 1.0732e-2 |  | 5.1021e-2 |  |
| 11 | 121 | 1.0948e-2 | 2.013% | 5.2308e-2 | 2.522% |
| 16 | 256 | 1.0975e-2 | 0.248% | 5.2550e-2 | 0.463% |
| 21 (study mesh) | 441 | 1.0994e-2 | 0.165% | 5.2635e-2 | 0.162% |
| 26 | 676 | 1.0997e-2 | 0.029% | 5.2675e-2 | 0.075% |
| 31 | 961 | 1.1002e-2 | 0.048% |  | 0.041% |
| 41 | 1681 | 1.1005e-2 | 0.027% |  | 0.040% |
| 51 | 2601 | 1.1006e-2 | 0.013% |  | 0.019% |

Table 2. B2 x Neo-Hookean mesh convergence (N=6-51)

| N | Nodes | Max ¦u¦ | Δ vs. prev. | Strain energy U | Δ vs. prev. |
| --- | --- | --- | --- | --- | --- |
| 6 | 36 | 3.8375e-3 |  |  |  |
| 11 | 121 | 3.8901e-3 | 1.373% |  | 0.463% |
| 16 | 256 | 3.8930e-3 | 0.073% |  | 0.120% |
| 21 (study mesh) | 441 | 3.8989e-3 | 0.152% |  | 0.051% |
| 26 | 676 | 3.8990e-3 | 0.003% |  | 0.028% |
| 31 | 961 | 3.9010e-3 | 0.052% |  | 0.017% |
| 41 | 1681 | 3.9019e-3 | 0.023% |  | 0.019% |
| 51 | 2601 | 3.9024e-3 | 0.012% |  | 0.010% |

Table 1a. B1 x Mooney-Rivlin mesh convergence (N=6-51)

| N | Nodes | Max ¦u¦ | Δ vs. prev. | Strain energy U | Δ vs. prev. |
| --- | --- | --- | --- | --- | --- |
| 6 | 36 | 5.8389e-3 |  |  |  |
| 11 | 121 | 5.9131e-3 | 1.270% |  | 0.272% |
| 16 | 256 | 5.9132e-3 | 0.002% |  | 0.067% |
| 21 (study mesh) | 441 | 5.9214e-3 | 0.139% |  | 0.028% |
| 26 | 676 | 5.9205e-3 | 0.015% |  | 0.015% |
| 31 | 961 | 5.9233e-3 | 0.048% |  | 0.009% |
| 41 | 1681 | 5.9241e-3 | 0.013% |  | 0.010% |
| 51 | 2601 | 5.9244e-3 | 0.007% |  | 0.005% |

Table 1b. B1 x Arruda-Boyce mesh convergence (N=6-51)

| N | Nodes | Max ¦u¦ | Δ vs. prev. | Strain energy U | Δ vs. prev. |
| --- | --- | --- | --- | --- | --- |
| 6 | 36 | 7.0981e-3 |  |  |  |
| 11 | 121 | 7.2420e-3 | 2.027% |  | 2.554% |
| 16 | 256 | 7.2604e-3 | 0.254% |  | 0.469% |
| 21 (study mesh) | 441 | 7.2724e-3 | 0.166% |  | 0.164% |
| 26 | 676 | 7.2746e-3 | 0.031% |  | 0.076% |
| 31 | 961 | 7.2781e-3 | 0.048% |  | 0.041% |
| 41 | 1681 | 7.2801e-3 | 0.028% |  | 0.041% |
| 51 | 2601 | 7.2811e-3 | 0.013% |  | 0.019% |

Table 2a. B2 x Mooney-Rivlin mesh convergence (N=6-51)

| N | Nodes | Max ¦u¦ | Δ vs. prev. | Strain energy U | Δ vs. prev. |
| --- | --- | --- | --- | --- | --- |
| 6 | 36 | 1.0124e-2 |  |  |  |
| 11 | 121 | 1.0327e-2 | 1.999% |  | 2.432% |
| 16 | 256 | 1.0351e-2 | 0.233% |  | 0.446% |
| 21 (study mesh) | 441 | 1.0368e-2 | 0.167% |  | 0.156% |
| 26 | 676 | 1.0371e-2 | 0.026% |  | 0.072% |
| 31 | 961 | 1.0376e-2 | 0.049% |  | 0.039% |
| 41 | 1681 | 1.0379e-2 | 0.026% |  | 0.039% |
| 51 | 2601 | 1.0380e-2 | 0.012% |  | 0.018% |

Table 2b. B2 x Arruda-Boyce mesh convergence (N=6-51)

Figure 17. Mesh convergence: strain-energy change per refinement vs. N, B1 and B2, all three materials (Tables 1/1a/1b/2/2a/2b).

| N | n_dof | L2 rel. | H1 rel. | Energy (tangent) rel. | Wall-clock |
| --- | --- | --- | --- | --- | --- |
| 51 | 5,202 | 4.93e-4 | 1.76e-2 | 1.52e-2 | 226.6 s |
| 101 | 20,402 | 1.87e-4 | 1.10e-2 | 9.19e-3 | 445.3 s |
| 201 | 80,802 | 6.99e-5 | 6.30e-3 | 5.50e-3 | 883.9 s |
| 401 | 321,602 | 2.50e-5 | 3.59e-3 | 3.14e-3 | 2615.8 s |
| 701 | 982,802 | 1.01e-5 | 2.85e-3 | 1.91e-3 | 7205.4 s |
| 1001 | 2,004,002 | 5.15e-6 | 1.85e-3 | 1.27e-3 | 17314.8 s |
| 1401 | 3,925,602 | 2.29e-6 | 1.63e-3 | 7.82e-4 | 27257.4 s |

Table 6a. B1 x Neo-Hookean, Q4 vs ~10M-DOF reference. Every row is now solved with the geometric multigrid preconditioner (see the CG-convergence note below), so N=401-1401 are genuinely converged, not the CG-capped solves an earlier revision used. Fitted rates: L2 p=1.57, H1 p=0.73, energy p=0.86 -- within 0.01 of the earlier CG-capped fit; only the wall-clock column changes substantially.

Figure 30. Mesh-convergence field grid (B1, Neo-Hookean): predicted |u| across seven resolutions, N=51-1401, one converged solution each (Table 6a).

| Element order | L2 rate (p) | H1 rate (p) | Energy rate (p) |
| --- | --- | --- | --- |
| Q4 | 1.39 | 0.72 | 0.71 |
| Q9 | 1.57 | 0.76 | 0.75 |

Table 6b. Fitted convergence rates, Q4 vs. Q9 (B1 × Neo-Hookean, same 6 shared resolutions N=6,11,16,21,31,41).

| Norm | Rel. diff. (Q4-domain) | Rel. diff. (Q9-domain) | Advisor target | Verdict |
| --- | --- | --- | --- | --- |
| L2 | 2.06e-6 | 2.06e-6 | <1e-5 | PASS |
| H1-seminorm | 1.11e-3 | 1.15e-3 | <1e-5 | FAIL |
| Tangent-energy | 9.61e-4 | 3.10e-4 | <1e-5 | FAIL |

Table 6c. Direct Q4-vs-Q9 fine-solution comparison (B1 × Neo-Hookean, N=2236, ~10M/40M DOF) against the advisor’s explicit <1e-5 criterion.

## 2. GPU memory: allocated / reserved / device peak

All three quantities measured for all 6 cases, out of the GPU’s total reported device memory.

| Case | Peak allocated (MB) | Peak reserved (MB) | Peak device / nvidia-smi-equiv. (MB) | Device total (MB) |
| --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 425.05 | 679.48 | 1226.77 | 85,094.83 |
| B1 × Mooney-Rivlin | 425.05 | 677.38 | 1224.67 | 85,094.83 |
| B1 × Arruda-Boyce | 425.05 | 679.48 | 1228.87 | 85,094.83 |
| B2 × Neo-Hookean | 425.05 | 679.48 | 1226.77 | 85,094.83 |
| B2 × Mooney-Rivlin | 425.05 | 677.38 | 1224.67 | 85,094.83 |
| B2 × Arruda-Boyce | 425.05 | 679.48 | 1228.87 | 85,094.83 |

Table 8. GPU memory: allocated / reserved / device peak

## 3. Training cost vs. native FEM, and inference latency

Native (CPU) FEM cost measured directly per case; training cost, speed-up, and inference latency computed for all 6 cases.

| Case | Linear solver | Newton tol. | Newton iters. (med., total) | Load steps | Assembly (s) | Solve (s) | Total (s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B1 × Neo-Hookean (CPU) | scipy sparse LU (spsolve) | 1e-7 | 30.0 | 10 | 25.343 | 0.082 | 25.432 |
| B2 × Neo-Hookean (CPU) | scipy sparse LU (spsolve) | 1e-7 | 30.0 | 10 | 25.809 | 0.089 | 25.909 |
| B1 × Mooney-Rivlin (CPU) | scipy sparse LU (spsolve) | 1e-7 | 30.0 | 10 | 53.647 | 0.082 | 53.735 |
| B1 × Arruda-Boyce (CPU) | scipy sparse LU (spsolve) | 1e-7 | 30.0 | 10 | 52.452 | 0.083 | 52.542 |
| B2 × Mooney-Rivlin (CPU) | scipy sparse LU (spsolve) | 1e-7 | 30.0 | 10 | 61.613 | 0.089 | 61.712 |
| B2 × Arruda-Boyce (CPU) | scipy sparse LU (spsolve) | 1e-7 | 30.0 | 10 | 60.185 | 0.090 | 60.285 |

Table 4a. Native (CPU) FEM cost per sample, all 6 cases

| Material | Assembly FLOPs | Solve FLOPs |
| --- | --- | --- |
| Neo-Hookean | ≈5.88×10⁷ | ≈7.9×10⁵ |
| Mooney-Rivlin | ≈9.72×10⁷ | ≈7.9×10⁵ |
| Arruda-Boyce | ≈9.72×10⁷ | ≈7.9×10⁵ |

Table 4b. FLOPs per sample for the CPU direct FEM solver (hand-counted per element per Gauss point, not profiler-measured). Assembly cost is material-dependent the autodiff-derived tangents of Mooney-Rivlin and Arruda-Boyce cost ≈1.65× Neo-Hookean’s; the solve count is geometry/material-independent at this mesh.

| Case | Opt. steps | (ms) | (s) | Native FEM (s) | Speed-up | Inference (ms/sample) |
| --- | --- | --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 75,000 | 4.79 | 3.59 | 25.432 | 7.08× | 4.586 |
| B1 × Mooney-Rivlin | 70,000 | 4.97 | 3.48 | 53.735 | 15.44× | 4.693 |
| B1 × Arruda-Boyce | 67,500 | 5.29 | 3.57 | 52.542 | 14.72× | 4.745 |
| B2 × Neo-Hookean | 840,000 | 38.39 | 40.31 | 25.909 | 0.64× | 4.809 |
| B2 × Mooney-Rivlin | 840,000 | 40.67 | 42.71 | 61.712 | 1.44× | 4.908 |
| B2 × Arruda-Boyce | 580,000 | 42.84 | 31.06 | 60.285 | 1.94× | 4.984 |

Table 7. Training cost vs. native FEM, and inference latency

| Case | Break-even (new samples) |
| --- | --- |
| B1 × Neo-Hookean | 113 |
| B1 × Mooney-Rivlin | 52 |
| B1 × Arruda-Boyce | 54 |
| B2 × Neo-Hookean | 1,245 |
| B2 × Mooney-Rivlin | 554 |
| B2 × Arruda-Boyce | 412 |

Table 7b. Break-even point per case: number of future inferences after which training + inference cost drops below repeated native FEM solves.

## 4. GPU-native finite-element solver

New batched GPU solver validated to machine precision against the CPU reference, then timed.

| Case | max¦u_cpu¦ | max abs. diff. | mean rel. err. | max rel. err. | Verdict |
| --- | --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 2.150e-3 | 2.60e-16 | 1.32e-13 | 7.23e-13 | PASS |
| B1 × Mooney-Rivlin | 1.404e-3 | 2.23e-16 | 2.14e-13 | 1.41e-12 | PASS |
| B1 × Arruda-Boyce | 2.134e-3 | 2.46e-16 | 1.40e-13 | 9.02e-13 | PASS |
| B2 × Neo-Hookean | 1.910e-2 | 4.13e-16 | 1.08e-14 | 3.51e-14 | PASS |
| B2 × Mooney-Rivlin | 1.251e-2 | 2.75e-16 | 1.42e-14 | 4.23e-14 | PASS |
| B2 × Arruda-Boyce | 1.829e-2 | 4.70e-16 | 1.62e-14 | 4.82e-14 | PASS |

Table 9. GPU-native solver vs. CPU reference (correctness)

| Case | bs=1 (ms/sample) | bs=8 (ms/sample) | bs=32 (ms/sample) | bs=128 (ms/sample) | Speed-up @ bs=128 |
| --- | --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 1651.6 | 477.9 | 381.3 | 354.6 | 71.72× |
| B1 × Mooney-Rivlin | 2047.7 | 492.4 | 388.3 | 359.7 | 149.4× |
| B1 × Arruda-Boyce | 2524.6 | 525.5 | 410.9 | 378.3 | 138.9× |
| B2 × Neo-Hookean | 1665.9 | 479.0 | 381.3 | 354.7 | 73.05× |
| B2 × Mooney-Rivlin | 2010.5 | 492.8 | 389.1 | 359.8 | 171.5× |
| B2 × Arruda-Boyce | 2612.1 | 529.3 | 411.1 | 378.5 | 159.3× |

Table 10. GPU-native FEM timing, all 6 cases

Transolver re-benchmarked at the same batch sizes as the GPU FEM solver above, so the two are directly comparable.

| Case | bs=1 | bs=8 | bs=32 | bs=128 |
| --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 4.582 | 0.573 | 0.336 | 0.292 |
| B1 × Mooney-Rivlin | 4.832 | 0.607 | 0.336 | 0.292 |
| B1 × Arruda-Boyce | 4.753 | 0.599 | 0.336 | 0.292 |
| B2 × Neo-Hookean | 4.680 | 0.585 | 0.336 | 0.291 |
| B2 × Mooney-Rivlin | 4.714 | 0.594 | 0.336 | 0.291 |
| B2 × Arruda-Boyce | 4.799 | 0.602 | 0.336 | 0.292 |

Table 10a. Transolver inference latency, ms/sample

Operator vs. GPU FEM, both at the same batch size (Table 10 ÷ Table 10a).

| Case | bs=1 | bs=8 | bs=32 | bs=128 |
| --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 360× | 833× | 1,134× | 1,215× |
| B1 × Mooney-Rivlin | 424× | 812× | 1,154× | 1,232× |
| B1 × Arruda-Boyce | 531× | 878× | 1,221× | 1,297× |
| B2 × Neo-Hookean | 356× | 819× | 1,135× | 1,217× |
| B2 × Mooney-Rivlin | 426× | 830× | 1,158× | 1,235× |
| B2 × Arruda-Boyce | 544× | 879× | 1,223× | 1,297× |

Table 10b. Operator speed-up over GPU-native FEM, matched batch sizes. Unmatched (FEM at bs=128 vs. operator at bs=1) gives 73–80×.

Figure 18. GPU-native FEM solver vs. trained-operator latency across batch sizes, hardware-matched (Tables 10/10a/10b).

New problems that must be solved before training costs (Table 7) is repaid by the per-sample saving over the GPU solver.

| Case | bs=1 | bs=8 | bs=32 | bs=128 |
| --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 1,745 | 6,021 | 7,543 | 8,112 |
| B1 × Mooney-Rivlin | 1,363 | 5,663 | 7,179 | 7,751 |
| B1 × Arruda-Boyce | 1,133 | 5,441 | 6,956 | 7,554 |
| B2 × Neo-Hookean | 19,410 | 67,391 | 84,627 | 90,990 |
| B2 × Mooney-Rivlin | 17,033 | 69,404 | 87,884 | 95,038 |
| B2 × Arruda-Boyce | 9,530 | 46,993 | 60,490 | 65,698 |

Table 10c. Break-even vs. GPU-native FEM, in problem instances. Range 1,133–95,038: the assumed batch size matters more than the case. Against the CPU baseline it is 52–1,245.

Break-even against both baselines together. CPU column from Tables 4a and 7; GPU columns are Table 10c.

| Case | vs. CPU FEM | vs. GPU FEM, bs=1 | bs=8 | bs=32 | bs=128 |
| --- | --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 113 | 1,745 | 6,021 | 7,543 | 8,112 |
| B1 × Mooney-Rivlin | 52 | 1,363 | 5,663 | 7,179 | 7,751 |
| B1 × Arruda-Boyce | 54 | 1,133 | 5,441 | 6,956 | 7,554 |
| B2 × Neo-Hookean | 1,245 | 19,410 | 67,391 | 84,627 | 90,990 |
| B2 × Mooney-Rivlin | 554 | 17,033 | 69,404 | 87,884 | 95,038 |
| B2 × Arruda-Boyce | 412 | 9,530 | 46,993 | 60,490 | 65,698 |

Table 10d. Break-even in new problem instances, both baselines. 52–1245 against CPU; 1,133–95,038 against GPU, of which 1,133–19,410 at batch size 1, the deployment case.

Figure 19. Break-even, in new problem instances, against CPU-FEM and GPU-native FEM baselines (Table 10d, reusing Table 10c's own GPU-FEM columns).

Scaling study: the matrix-free Newton-CG solver (not the dense batched one above, which cannot reach these sizes a 2M×2M FP64 tangent is ~32 TB), eight resolutions on one A100-SXM4-80GB, FP64, 10 load steps.

| N | DOF | Solve | µs/DOF | Peak GPU (MB) | resid/precond/CG (%) |
| --- | --- | --- | --- | --- | --- |
| 101 | 20,402 | 6.6 min | 19,410 | — | 0.3/0.3/99.4 |
| 201 | 80,802 | 13.3 min | 9,876 | — | 0.1/0.1/99.8 |
| 301 | 181,202 | 20.4 min | 6,755 | — | 0.0/0.1/99.9 |
| 401 | 321,602 | 27.7 min | 5,168 | — | 0.0/0.1/99.9 |
| 501 | 502,002 | 26.9 min | 3,219 | 1,123 | — |
| 701 | 982,802 | 1.2 h | 4,566 | 1,568 | — |
| 1001 | 2,004,002 | 3.4 h | 6,073 | 2,035 | — |
| 1401 | 3,925,602 | 11.0 h | 10,125 | 3,280 | — |

Table 20. Largest case: 3,925,602 DOF in 11.0 h using 3,280 MB of 80 GB (~4%). Memory is not the constraint, and the memory model held out of sample to 2.4%.

Figure 20. GPU-native matrix-free solver scaling: wall-clock time and CG iteration count vs. problem size (Table 20/20a/20c).

The timings in that table need a caveat the report now states in full. The solver's own counters show CG did not converge at N ≥ 401: it hit its 2,000-iteration cap on EVERY Newton step, so those wall clocks are the cost of a truncated budget rather than of a converged solve. Accuracy is unaffected Newton's test is on the absolute residual, and its counts stay far below their limit but the cost analysis changes. The three resolutions where CG did converge fix the real requirement at 5.01 × N iterations per Newton solve, the textbook O(1/h) rate, which means N=1401 completed only 28% of the iterations it needed.

What survives cleanly and is the better result: dividing CG time by iteration count gives 76, 76, 76 nanoseconds per iteration per degree of freedom at N = 701, 1001 and 1401 flat to 0.7% across a fourfold size change. The matrix-free product is O(DOF) in time, measured rather than fitted. The previously reported DOF^1.54 exponent did not measure the solver's scaling: those runs each ran a constant 2,000 iterations per Newton step and what grew was the Newton count, which grows only because a truncated CG returns an inexact direction.

That caveat has since been closed. N = 501 and 701 were re-run with the CG cap raised from 2,000 to 8,000 and nothing else changed. CG converged at both zero capped solves and the O(N) law, fitted on meshes no larger than N = 301, predicted 2,511 and 3,513 iterations per Newton solve against the 2,520.8 and 3,528.1 measured, 0.4% at both. The prediction was printed before the runs were launched. Truncation does cost Newton steps as expected at N = 701 the count fell from 30 to 20 but not enough to pay for the missing iterations: Table 20 UNDERSTATES the converged cost, by 28% at N = 501 (1,616 s → 2,063 s) and 18% at N = 701 (4,487 s → 5,286 s). The per-CG-iteration cost agrees between the truncated and converged runs to 1.3%, which is two independent measurements of the quantity the O(DOF) claim rests on. N = 1001 and 1401 were not re-run; the same model puts them at +25% and +5%, and those two are predictions.

That prediction is now superseded by a real fix, not another cap raise. A 2x2 block-Jacobi preconditioner was tried first and measured to do nothing (cg_failures unchanged at N=401/701). A geometric multigrid V-cycle was built instead -- validated on small CPU meshes first (matches plain Jacobi to 1e-12, advantage growing 3.0x to 9.7x with size) -- and run for real at all four resolutions that hit the cap.

| N | DOF | CG per Newton solve | ms per CG iteration | Solve time |
| --- | --- | --- | --- | --- |
| 401 | 321,602 | 153.3 | 853.2 | 43.6 min |
| 701 | 982,802 | 135.5 | 2,658.8 | 2.00 h |
| 1001 | 2,004,002 | 260.9 | 3,317.7 | 4.81 h |
| 1401 | 3,925,602 | 318.9 | 4,274.3 | 7.57 h |

Every one converges within the ORIGINAL 2,000-iteration cap (0 of 20 hit it), including N=1001 and N=1401 which the cap-raise approach could only predict. Multigrid needs 19-26x fewer iterations per Newton solve than Jacobi's O(N) law predicts (135.5/261.0/318.9 against 3,513/~5,016/~7,020 at N=701/1001/1401), flat and sub-linear where Jacobi grows without bound -- the real signature of working multigrid. Each iteration costs more, though: against Table 20b's directly measured N=701 (5,286 s), multigrid is 36% slower; against the same law's unmeasured, predicted N=1001 (~15,349 s), 13% slower; against its unmeasured, predicted N=1401 (~41,675 s), multigrid is 35% FASTER. The crossover -- a net loss at 701, roughly break-even at 1001, a clear win at 1401 -- is multigrid's flatter iteration count starting to outrun its larger fixed overhead as N grows, which is the reason to prefer it going forward even though it is not a clean win everywhere tested. Caveat: N=701/1001/1401's coarsest mesh level (~32,000-62,000 free DOF) is too large to factor exactly like N=401's own ~1,300-DOF coarsest level, so those three solve their coarsest level approximately (a capped 50-iteration CG pass) -- a standard multigrid variant, and a plausible partial explanation for the smaller advantage at 701/1001.

µs/DOF is U-shaped: falls 6.0× from 19,410 at N=101 to 3,219 at N=501, then rises 3.1× to 10,125 at N=1401. Below the minimum the GPU is under-occupied (launch overhead, same effect as bs=1 in Table 10a); above it the CG iteration count grows because the tangent's condition number scales as 1/h² and Jacobi only partly offsets it. Cost on the large branch fits DOF^1.54. So: O(DOF) in memory, not in time and the solver has an operating range rather than simply degrading.

Memory: 818 MB fixed + 607 MB per million DOF, a line through N=501 and N=1001 built before N=1401 ran. It predicted 3,201 MB; the measured peak was 3,280 MB 2.4% out of sample. Caveat: N=701 sits 10% above that line, so the linearity is good to about ten per cent, not better.

Cost breakdown: explicit assembly (residual + Jacobi preconditioner) 0.1–0.6%, CG 99.4–99.9%. This superficially confirms Timon's expectation that assembly should be minimal and should NOT be quoted that way: matrix-free means every CG iteration IS a Hessian-vector product, i.e. an assembly-like pass over all elements. The assembly did not get cheap; it moved inside CG where this instrumentation cannot see it. The clean split exists only for assemble-once-and-factories solvers on the CPU reference (Table 4a) assembly outweighs the sparse solve by 290–692×.

GPU-FEM vs. torch-fem (item #13, R7.1): direct comparison against an established, GPU-accelerated FEM library, requested explicitly with no preference between torch-fem and TensorMesh. Same mesh/material/BCs/load as Table 20c, same four N. Correctness confirmed first at small N on CPU (2-5e-6 relative displacement difference, inside torch-fem's float32 precision).

| N | Ours (mgv) | torch-fem | Speedup (ours slower by) | torch-fem peak mem |
| --- | --- | --- | --- | --- |
| 401 | 43.6 min | 6.3 s | 418x | 3,293 MB |
| 701 | 2.00 h | 11.0 s | 655x | 9,670 MB |
| 1001 | 4.81 h | 14.5 s | 1,197x | 19,499 MB |
| 1401 | 7.57 h | 29.1 s | 936x | 38,064 MB |

torch-fem wins wall-clock by a large, non-monotonic margin (peaks at N=1001, dips at N=1401). Two caveats, not hidden: (1) torch-fem forced to float32/loose tolerance (rtol=atol=1e-3) vs. our float64/tight (1e-8) — likely explains much of the gap; (2) "ours" resumed from item #4's checkpoint rather than solving fresh, so no real "ours" peak-memory number exists to set against torch-fem's measured one (Omar's own call: not worth the extra GPU-hours a fresh solve would cost). Not read as a defect: torch-fem assembles a sparse tangent once and factorizes it; our solver never forms one at all, by design, so it reaches DOF counts (the ~10M-DOF reference this report's own Table 6a depends on) an assembled approach could not hold in GPU memory. Timon's round-8 review already predicted this exact trade-off (TensorMesh "presumably assembles explicitly once and factorizes", vs. our solver's per-CG-iteration element work) — Table 20d/this result is a direct empirical confirmation of that same concern.

Figure 21. GPU-native solver vs. torch-fem: solve time and torch-fem peak memory across resolutions (Table 20d).

## 5. Out-of-distribution generalization

Mitigation tested rather than proposed. Section 8.5 named normalizing the material channel as the cheapest candidate fix. It was run: the identical protocol on the identical dataset with input normalization as the only change, then both sweeps repeated on a shared cache of FEM references. It does not work.

| Factor | k | Raw | Normalized | Raw × | Norm × |
| --- | --- | --- | --- | --- | --- |
| material | 0.5 | 0.1409 | 0.1700 | 1.63× | 1.84× |
| material | 1.0 | 0.2073 | 0.2577 | 2.39× | 2.79× |
| material | 1.5 | 0.2763 | 0.3385 | 3.19× | 3.66× |
| material | 2.0 | 0.3486 | 0.3973 | 4.02× | 4.30× |
| material | 2.5 | 0.4265 | 0.4030 | 4.92× | 4.36× |
| material | 3.0 | 0.5112 | 0.3565 | 5.90× | 3.85× |
| loading | 0.5 | 0.0869 | 0.0992 | 1.00× | 1.07× |
| loading | 1.0 | 0.0888 | 0.1025 | 1.02× | 1.11× |
| loading | 1.5 | 0.0897 | 0.1043 | 1.03× | 1.13× |
| loading | 2.0 | 0.0871 | 0.1040 | 1.00× | 1.12× |
| loading | 2.5 | 0.0862 | 0.1015 | 0.99× | 1.10× |
| loading | 3.0 | 0.0927 | 0.1274 | 1.07× | 1.38× |
| both | 0.5 | 0.1427 | 0.1747 | 1.65× | 1.89× |
| both | 1.0 | 0.2066 | 0.2459 | 2.38× | 2.66× |
| both | 1.5 | 0.2682 | 0.2979 | 3.09× | 3.22× |
| both | 2.0 | 0.3294 | 0.3293 | 3.80× | 3.56× |
| both | 2.5 | 0.3957 | 0.3540 | 4.56× | 3.83× |
| both | 3.0 | 0.4601 | 0.4064 | 5.31× | 4.39× |

Table 19a. Both models over the same sweep. Each "×" divides by that model's own in-distribution error (0.0867 raw, 0.0925 normalized).

Why it is a negative result and not a win: normalization costs 6.7% in distribution confirmed on two independent metrics and despite 40% MORE optimizer steps and makes the absolute error worse in 13 of 18 shifted cells, everyone at or below 1.5σ among them. It’s one striking gain, the material ratio at 3σ, sits on a curve that peaks at k = 2.5 and then FALLS: an error that stops growing as the shift grows is a prediction collapsing toward something input-independent, not extrapolation. The loading rows are the control and stay flat for both models, so the sensitivity did not move. The §8.5 mechanism stands standardizing is affine, so a shifted E is still outside the trained range. The untested candidate that remains is predicting a stiffness-scaled quantity instead of the displacement.

All 6 existing checkpoints evaluated on a shifted material/load distribution, no retraining.

| Case | In-distribution val. err. | OOD val. err. | Degradation factor (OOD / ID) |
| --- | --- | --- | --- |
| B1 × Neo-Hookean | 0.0959 | 0.3939 | 4.11× |
| B1 × Mooney-Rivlin | 0.1121 | 0.4731 | 4.22× |
| B1 × Arruda-Boyce | 0.1070 | 0.4927 | 4.61× |
| B2 × Neo-Hookean | 0.0911 | 0.4825 | 5.30× |
| B2 × Mooney-Rivlin | 0.0728 | 0.4060 | 5.58× |
| B2 × Arruda-Boyce | 0.0981 | 0.3868 | 3.94× |

Table 11. In-distribution vs. out-of-distribution error

Figure 22. In-distribution vs. out-of-distribution validation error and degradation factor, all six cases (Table 11).

Shift in units of the training distribution’s own std, one factor at a time, 10 held-out samples per point, B1 × Neo-Hookean, N=21. Same per-component metric as Table 11.

| k (σ) | Loading | × | Material | × | Both | × |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0 (ID) | 0.0867 | — | — | — | — | — |
| 0.5 | 0.0869 | 1.00× | 0.1409 | 1.63× | 0.1427 | 1.65× |
| 1.0 | 0.0888 | 1.02× | 0.2073 | 2.39× | 0.2066 | 2.38× |
| 1.5 | 0.0897 | 1.03× | 0.2763 | 3.19× | 0.2682 | 3.09× |
| 2.0 | 0.0871 | 1.00× | 0.3486 | 4.02× | 0.3294 | 3.80× |
| 2.5 | 0.0862 | 0.99× | 0.4265 | 4.92× | 0.3957 | 4.56× |
| 3.0 | 0.0927 | 1.07× | 0.5112 | 5.90× | 0.4601 | 5.31× |

Table 19. Loading causes no degradation anywhere (0.99–1.07× over the whole sweep); material causes all of it (1.63–5.90×). Table 11’s 4.11× for this case sits inside the combined column (3.80× at k=2.0, 4.56× at k=2.5). Smooth, no threshold. The two shifts partially cancel: “both” is 10% below “material” at k=3.

Figure 31. Out-of-distribution shift field grid (B1, Neo-Hookean): predicted |u| under material shift, k=0-3sigma (Tables 19/25).

Figure 32. Out-of-distribution shift field grid (B1, Mooney-Rivlin): predicted |u| under material shift, k=0-3sigma (Tables 19/25).

Figure 33. Out-of-distribution shift field grid (B1, Arruda-Boyce): predicted |u| under material shift, k=0-3sigma (Tables 19/25).

Figure 34. Out-of-distribution shift field grid (B2, Neo-Hookean): predicted |u| under material shift, k=0-3sigma (Tables 19/25).

Figure 35. Out-of-distribution shift field grid (B2, Mooney-Rivlin): predicted |u| under material shift, k=0-3sigma (Tables 19/25).

Figure 36. Out-of-distribution shift field grid (B2, Arruda-Boyce): predicted |u| under material shift, k=0-3sigma (Tables 19/25).

The same per-factor isolation has now been run for the other five geometry × material combinations (Table 25 in the report). Loading-shift degradation stays mild everywhere, 0.81×–1.35× at k=3σ, and is at or below baseline for two cases (B2 × Neo-Hookean 0.99×, B2 × Arruda-Boyce 0.81×). Material-shift degradation dominates every case, 2.27×–5.90× at k=3σ — an order of magnitude worse than loading throughout. Mooney-Rivlin is the most fragile material on both geometries (4.71× on B1, 5.47× on B2); Arruda-Boyce is the most robust (4.85× on B1, only 2.27× on B2). B2 × Arruda-Boyce is also the only case where the loading-shift degradation measures below baseline at EVERY σ tested, 0.79×–0.93×, not just at k=3 — the same sign at all six shift magnitudes, which rules out sampling noise.

Candidate future work on this: continual learning, which adapts a trained operator to new distributions incrementally instead of insisting on one fixed zero-shot model. Wang, Eshaghi, Zhuang, Rabczuk, and Liu, "Replay-Based Continual Learning for Physics-Informed Neural Operators" (arXiv:2605.04832), do exactly this — replay-and-distillation, no labeled data — on the same Transolver architecture used throughout this report. Not implemented or tested here; named as a candidate direction only.

## 6. Resolution invariance (zero-shot)

True zero-shot protocol (train once, evaluate on unseen resolutions vs. a common fine reference) done for 1 of 6 cases. The other 5 are running now.

| N | Neo-Hookean | Mooney-Rivlin | Arruda-Boyce |
| --- | --- | --- | --- |
| 13 | 0.0967 | 0.1064 | 0.1011 |
| 17 | 0.0791 | 0.0885 | 0.0832 |
| 25 | 0.0574 | 0.0691 | 0.0647 |
| 29 | 0.0521 | 0.0628 | 0.0597 |
| 37 | 0.0525 | 0.0541 | 0.0564 |
| 41 | 0.0562 | 0.0515 | 0.0575 |
| 49 | 0.0670 | 0.0504 | 0.0630 |

Table 12 (revised). Zero-shot resolution invariance, all three B1 materials. One checkpoint per material, trained JOINTLY at N = 21 and 33, evaluated with no retraining at seven unseen resolutions against a common N=101 reference, twenty realizations each. An earlier revision described the Neo-Hookean checkpoint as trained at N=21 alone; it was not, and the correction does not move any of its numbers.

Figure 37. Zero-shot resolution invariance field grid (B1, Neo-Hookean): predicted |u| at seven unseen resolutions from one trained checkpoint (Table 12).

Figure 23. Zero-shot resolution invariance: mean relative L2 error vs. test resolution, B1 and B2, all three materials (Table 12).

All three stay between 5.0% and 10.6% across every unseen resolution, with no retraining. But reading down the columns shows what one material could not: two of three reach their best near the training range and then get WORSE on the finest meshes Neo-Hookean bottoms at N = 29 and rises 28.7% by N = 49, Arruda-Boyce bottoms at N = 37 and rises 11.6%, while Mooney-Rivlin keeps improving to 0.0504. Zero-shot transfer to much finer meshes is not free and how much it costs is material-dependent.

B2 was excluded from Table 12 twice, and the second exclusion was our own mistake. First its sample caches were found to carry an applied load overstated by a mesh-dependent factor about 13× at N=21 against 21× at N=33 which for a study measuring transfer across meshes invalidates them outright. The caches were repaired and checked (one fixed pressure field now assembles to 0.007% across the two meshes) and all three cases retrained under the B1 protocol with the per-sample force normalization section 9.1 requires. They still reported best validation errors of 0.9986, 0.9752 and 1.0267 what predicting zero scores and that was written up as an unexplained failure on the curved geometry.

The cause was the criterion used to stop training, not the models. Validation used the per-component average, which divides each displacement component by its own magnitude. On B2 the per-sample ratio of the two component magnitudes averages 1.90 while the ratio of the averaged components is 0.90 a skewed distribution, so the mean of the per-sample ratios reports its tail and the resulting quantity rises over intervals where the field is in fact getting closer to the reference. Every B2 run exhausted its patience at its first or second validation event, epochs 25, 25 and 225, so every downstream B2 diagnosis the load, the ramp, the batch size, the functional, joint training was measuring a model trained for a few dozen epochs. B1 is unaffected: its two metrics differ by a stable 1.36–1.71× and all three B1 cases agree on which checkpoint is better, so every B1 figure stands as measured.

Re-running B2 × Neo-Hookean under exactly the Table 12 protocol, changing only the selection criterion to the combined norm ‖e‖/‖u‖ that Table 18 already uses, moves validation from 0.9986 to 0.0330 on the same metric that had condemned it, and to 0.0214 on the combined norm. Its best checkpoint is at epoch 2,750 against a previous best at epoch 25. Zero-shot at Table 12's seven unseen resolutions, per-component:

| N | before | after | combined norm, after |
| --- | --- | --- | --- |
| 13 | 0.8714 | 0.2314 | 0.2046 |
| 17 | 0.8718 | 0.1142 | 0.1091 |
| 25 | 0.8722 | 0.0720 | 0.0595 |
| 29 | 0.8723 | 0.0711 | 0.0655 |
| 37 | 0.8725 | 0.0808 | 0.0738 |
| 41 | 0.8726 | 0.1511 | 0.1432 |
| 49 | 0.8727 | 0.2691 | 0.2596 |

Read honestly, that is a working case and not a match for B1. In the middle of the range the two are comparable 0.0711 to 0.0808 at N = 25, 29 and 37 against B1's 0.0521 to 0.0691 but B2 degrades at both ends, 0.2314 at N = 13 and 0.2691 at N = 49, for a spread of 3.8× across the sweep where the worst B1 case spreads 2.1×. Training was at N = 21 and 33 and B2 falls away from those meshes in both directions. Its resolution invariance is real and weaker than B1's.

One correction to how the old numbers were described. The superseded model's error was flat across the mesh 0.8714 to 0.8727, 0.153% across a fourfold refinement, where the B1 columns move by 79%–111%. That flatness reads like the strongest resolution invariance in the report and is the opposite: a model whose output barely responds to its input gives nearly the same field, and so nearly the same error, on every mesh. Insensitivity and invariance are indistinguishable in that column. Flatness alone is not evidence of the property being tested.

Mooney-Rivlin and Arruda-Boyce were re-run immediately after, same protocol. Every B1/B2 × material combination now has a valid zero-shot result:

| N | Neo-Hookean | Mooney-Rivlin | Arruda-Boyce |
| --- | --- | --- | --- |
| 13 | 0.2314 | 0.1946 | 0.2565 |
| 17 | 0.1142 | 0.0790 | 0.1031 |
| 25 | 0.0720 | 0.0661 | 0.0940 |
| 29 | 0.0711 | 0.0730 | 0.0962 |
| 37 | 0.0808 | 0.1220 | 0.1383 |
| 41 | 0.1511 | 0.1997 | 0.2157 |
| 49 | 0.2691 | 0.3246 | 0.3402 |

All three B2 materials keep the pattern Neo-Hookean set: real resolution invariance, weaker than B1's. Each material's spread 4.91× Mooney-Rivlin, 3.78× Neo-Hookean, 3.62× Arruda-Boyce exceeds B1's worst case 2.11×, no exception. Arruda-Boyce is least accurate at 6 of 7 meshes but also most consistent (narrowest spread of the three) different properties, and this case separates them. Mooney-Rivlin and Neo-Hookean cross rather than one dominating: Mooney-Rivlin wins at every mesh coarser than the training resolutions (13, 17, 25) and loses at every mesh finer (37, 41, 49).

One more data point on the open patience question: Arruda-Boyce's early stop actually fired (the other two ran the full 4,000-epoch budget) best at epoch 1,450, stopped at 2,200, exactly best epoch + patience (15) × validate every (50). Patience 8 would have stopped this run before its actual best.

Separate from the above: a new study asks how the resolution of the TRAINING mesh itself (not the trained model’s own generalization) shapes zero-shot accuracy, for a data-driven (not physics-informed) operator. Two B1 × Neo-Hookean data-driven models were trained under the Table 21 protocol (800/200 samples, 75,000 steps, batch size 8), one labelled at N=13, the other at N=33, and both evaluated zero-shot at the same seven resolutions as Table 12. The coarse-trained model is most accurate at its own resolution (10.5% at N=13) but degrades to 25.9% at N=49, a 2.5× spread. The fine-trained model is worse at N=13 (13.7%) but its error DECREASES monotonically to 10.9% at N=49, staying inside a tight 10.9–13.7% band throughout. Training on a finer mesh trades some peak accuracy for materially better cross-resolution stability — the direct answer to this study’s own question.

Figure 38. DD-NO coarse-trained field grid (B1, Neo-Hookean): predicted |u| at seven unseen resolutions from a checkpoint trained on the COARSE mesh (Table 26).

Figure 39. DD-NO fine-trained field grid (B1, Neo-Hookean): predicted |u| at seven unseen resolutions from a checkpoint trained on the FINE mesh (Table 26).

Note on the "GOEE" / trust reference (separate from the above two items): the paper Timon sent has been confirmed directly with Omar as arXiv:2609.02982v1 (Cheng, Duruisseaux, Clauser et al., "Equation Recast for Canonical Operator Learning Across Parametric PDEs"). It is not goal-oriented error estimation in the adjoint/FEM sense used to name it — no dual problem, no computable QoI error bound. Its only trust-adjacent content is an informal, explicitly unverified heuristic: treating non-convergence of an internal fixed-point iteration as a runtime reliability flag for a prediction. Usable only for that narrower claim, as a candidate direction for future work, not as a method implemented anywhere in this report.

Note: this section covers the zero-shot study's own validated range (N up to 49). Its findings do NOT extrapolate to far larger meshes -- see Section 8's round-10 follow-up (Table 18-equivalent discussion) for the real, since-measured behavior at N=1401: a checkpoint-loading bug that was silently corrupting that specific measurement, the corrected accuracy (44.65% relative L2, degrading monotonically away from the training resolutions), and a multi-resolution retraining that reduced that error to 5.85% (7.6x) and changed the resolution-invariance picture substantially.

## 7. Error in physically important quantities beyond displacement

Trained operator scored on 50 held-out samples per case, N=21, against each sample's own same-mesh FE solution, in FP64. Mean over the 50 samples, worst sample in parentheses; percentages.

| Case | Displacement rel. L2 | L2 rel. | H1 semi-norm rel. | Tangent-energy rel. |
| --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 10.34 (23.4) | 6.58 (24.3) | 22.71 (42.8) | 13.78 (46.9) |
| B1 × Mooney-Rivlin | 11.57 (35.2) | 7.58 (36.9) | 24.20 (43.7) | 17.02 (53.6) |
| B1 × Arruda-Boyce | 11.71 (48.3) | 8.09 (43.7) | 23.02 (48.2) | 15.05 (44.8) |
| B2 × Neo-Hookean | 8.68 (27.1) | 7.48 (24.4) | 11.81 (28.1) | 11.54 (36.7) |
| B2 × Mooney-Rivlin | 7.21 (14.5) | 6.86 (18.8) | 10.47 (16.2) | 11.37 (34.3) |
| B2 × Arruda-Boyce | 10.47 (33.7) | 8.48 (24.4) | 13.30 (24.7) | 11.99 (34.3) |

Table 15. Displacement and integral-norm error, all six cases

First Piola-Kirchhoff stress, P =dWdF  by autodiff at the mesh's Gauss points. Means only; percentages.

| Case | ‖P‖F | P11 | P22 | P12 | P21 | peak ‖P‖ |
| --- | --- | --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 15.06 | 55.52 | 11.39 | 54.21 | 53.19 | 32.63 |
| B1 × Mooney-Rivlin | 18.14 | 78.75 | 12.86 | 57.59 | 57.04 | 19.87 |
| B1 × Arruda-Boyce | 15.53 | 53.53 | 12.71 | 65.35 | 64.36 | 47.53 |
| B2 × Neo-Hookean | 14.21 | 15.41 | 18.96 | 7.72 | 7.72 | 5.68 |
| B2 × Mooney-Rivlin | 14.23 | 17.64 | 16.45 | 7.74 | 7.76 | 5.38 |
| B2 × Arruda-Boyce | 13.32 | 15.83 | 15.67 | 9.04 | 9.04 | 5.99 |

Table 16. PK1 stress error. The P11 column on B1 and both shear columns on B1 divide by a near-zero reference read the ‖P‖F column, not those.

Reaction = internal force assembled from the same Gauss-point stresses on the constrained nodes. Means; percentages.

| Case | Constrained boundary | Resultant rel. err. | Nodal rel. L2 |
| --- | --- | --- | --- |
| B1 × Neo-Hookean | bottom edge ux, uyfixed | 4.66 | 7.10 |
| B1 × Mooney-Rivlin | bottom edge ux, uyfixed | 6.10 | 10.54 |
| B1 × Arruda-Boyce | bottom edge ux, uyfixed | 8.41 | 10.13 |
| B2 × Neo-Hookean | θ = 0 uyfixed | 9.38 | 12.47 |
| B2 × Neo-Hookean | θ =π2uxfixed | 7.82 | 10.16 |
| B2 × Mooney-Rivlin | θ = 0 uyfixed | 10.78 | 12.86 |
| B2 × Mooney-Rivlin | θ =π2uxfixed | 12.61 | 12.43 |
| B2 × Arruda-Boyce | θ = 0 uyfixed | 6.26 | 9.12 |
| B2 × Arruda-Boyce | θ =π2uxfixed | 7.17 | 10.81 |

Table 17. Reaction-force error. B2 is constrained on two symmetry edges and contributes a row for each.

Figure 24. Trained-operator displacement, stress, and reaction-force error, all six cases (Tables 15/16/17).

## 8. Accuracy/cost Pareto: operator vs. FEM, all three B1 materials

Both sides on the same 20 instances, same N=101 fine reference, same device, batch size 1. FEM cost is the CPU reference solver. Error is the combined relative L2 over both components NOT Table 12's per-component average, and drawn from a different seed range, so this column is not comparable with Table 12.

| N | Nodes | FEM rel. L2 (%) | FEM cost (s) | Operator rel. L2 (%) | Operator cost (ms) | Speed-up |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 169 | 0.608 | 8.9 | 6.76 | 5.491 | 1,630× |
| 17 | 289 | 0.383 | 16.0 | 5.93 | 5.517 | 2,898× |
| 21 | 441 | 0.270 | 24.9 | 5.12 | 4.584 | 5,432× |
| 25 | 625 | 0.203 | 36.1 | 4.43 | 5.542 | 6,513× |
| 29 | 841 | 0.156 | 49.1 | 3.93 | 5.508 | 8,906× |
| 33 | 1,089 | 0.124 | 63.6 | 3.70 | 5.530 | 11,502× |
| 37 | 1,369 | 0.101 | 81.0 | 3.69 | 5.563 | 14,562× |
| 41 | 1,681 | 0.083 | 99.3 | 3.84 | 5.550 | 17,895× |
| 49 | 2,401 | 0.059 | 142.6 | 4.44 | 5.555 | 25,676× |

Table 18. Accuracy and cost at nine resolutions. FEM at its course (N=13) is 0.608%, 6.1× more accurate than the operator at its best (3.69% at N=37); the two branches do not overlap. Operator cost is flat in mesh size (4.584–5.563 ms over a 14× node increase) while FEM grows super linearly, so the speed-up climbs 1,630× → 25,676×. Run twice: errors identical to every digit, timings 2.9× apart from Colab instances; the run consistent with Table 10a is reported. One of six cases. Caveat (2026-09-10): this flatness holds only up to N=49 (2,401 nodes, this sweep's own range). Measured directly at N=1401 (1,962,801 nodes), inference cost rises to 2,286.7 ms/sample -- ~500x higher, a sublinear-but-real ~n^0.74 scaling, not flat -- though still 59x faster than torch-fem's own matched-precision solve there (133.83s). Accuracy at N=1401 has since been checked directly (round-10 follow-up, below the round-9 section).

Table 18a. Mooney-Rivlin: operator error falls monotonically to N=49 (3.72%), no minimum inside the range unlike Neo-Hookean. Speed-up 3,574×–56,355×, ~2.2× larger than Neo-Hookean's throughout (2.1–2.4× costlier CPU assembly, Table 4a).

| N | Nodes | FEM rel. L2 (%) | FEM cost (s) | Operator rel. L2 (%) | Operator cost (ms) | Speed-up |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 169 | 0.624 | 20.4 | 8.87 | 5.701 | 3,574× |
| 17 | 289 | 0.395 | 36.4 | 7.69 | 5.512 | 6,600× |
| 21 | 441 | 0.280 | 56.9 | 6.81 | 5.521 | 10,310× |
| 25 | 625 | 0.210 | 81.7 | 6.04 | 5.546 | 14,738× |
| 29 | 841 | 0.162 | 111.4 | 5.35 | 5.517 | 20,192× |
| 33 | 1,089 | 0.129 | 145.2 | 4.77 | 5.576 | 26,041× |
| 37 | 1,369 | 0.105 | 183.9 | 4.30 | 5.588 | 32,914× |
| 41 | 1,681 | 0.087 | 227.9 | 3.95 | 5.586 | 40,799× |
| 49 | 2,401 | 0.062 | 327.6 | 3.72 | 5.814 | 56,355× |

Table 18b. Arruda-Boyce: bottoms at N=37 (3.53%) then rises to N=49 (4.27%) same shape as Neo-Hookean; Mooney-Rivlin is the exception among the three. Speed-up 3,452×–54,731×, between the other two. No material, at any mesh, has the operator matching even the cheapest FEM solve on accuracy.

Table 18c. B2 × Neo-Hookean (first of three B2 materials to finish). Speed-up 1,738×–27,392×.

| N | Nodes | FEM rel. L2 (%) | FEM cost (s) | Operator rel. L2 (%) | Operator cost (ms) | Speed-up |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 169 | 1.944 | 10.1 | 19.86 | 5.815 | 1,738× |
| 17 | 289 | 1.121 | 18.0 | 10.51 | 5.787 | 3,115× |
| 21 | 441 | 0.728 | 27.9 | 1.79 | 6.193 | 4,508× |
| 25 | 625 | 0.519 | 40.0 | 4.84 | 5.714 | 7,005× |
| 29 | 841 | 0.390 | 54.6 | 6.78 | 5.801 | 9,405× |
| 33 | 1,089 | 0.306 | 71.7 | 1.86 | 5.875 | 12,206× |
| 37 | 1,369 | 0.248 | 90.9 | 6.81 | 5.877 | 15,464× |
| 41 | 1,681 | 0.204 | 111.8 | 13.23 | 5.980 | 18,704× |
| 49 | 2,401 | 0.152 | 161.6 | 24.05 | 5.900 | 27,392× |

Unlike every B1 curve, this one is not smooth in N: sharp local minima at N=21 and N=33 the two resolutions this checkpoint was jointly trained on 4.30× and 3.65× lower than each point's neighbors. Table 18's B1 × Neo-Hookean curve, from a checkpoint trained at N=21 only, shows no such dip (1.01×, flat) candidate explanation (joint two-resolution training leaves visible anchor points), not established, since geometry and protocol differ at once between the two checkpoints compared.

Table 18d. B2 × Mooney-Rivlin (second of three B2 materials to finish). Speed-up 3,811×–60,651×.

| N | Nodes | FEM rel. L2 (%) | FEM cost (s) | Operator rel. L2 (%) | Operator cost (ms) | Speed-up |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 169 | 1.953 | 21.6 | 16.50 | 5.659 | 3,811× |
| 17 | 289 | 1.126 | 38.3 | 7.10 | 5.612 | 6,819× |
| 21 | 441 | 0.731 | 59.9 | 2.08 | 5.621 | 10,657× |
| 25 | 625 | 0.521 | 86.5 | 5.68 | 5.573 | 15,524× |
| 29 | 841 | 0.391 | 117.7 | 5.73 | 5.564 | 21,146× |
| 33 | 1,089 | 0.307 | 153.7 | 2.41 | 5.686 | 27,034× |
| 37 | 1,369 | 0.248 | 194.6 | 9.54 | 5.716 | 34,054× |
| 41 | 1,681 | 0.204 | 239.3 | 16.47 | 5.640 | 42,428× |
| 49 | 2,401 | 0.152 | 345.1 | 27.90 | 5.689 | 60,651× |

The anchoring effect replicates: local minima at N=21 and N=33 again, 3.07× and 3.17× lower than each point's neighbors smaller than Neo-Hookean's 4.30×/3.65× but the same shape, at the same two resolutions.

Table 18e. B2 × Arruda-Boyce (third and last B2 material). Speed-up 3,725×–59,234×.

| N | Nodes | FEM rel. L2 (%) | FEM cost (s) | Operator rel. L2 (%) | Operator cost (ms) | Speed-up |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 169 | 1.935 | 20.9 | 26.35 | 5.603 | 3,725× |
| 17 | 289 | 1.116 | 37.1 | 10.58 | 5.569 | 6,654× |
| 21 | 441 | 0.725 | 57.9 | 3.18 | 5.578 | 10,377× |
| 25 | 625 | 0.518 | 83.4 | 8.44 | 5.731 | 14,548× |
| 29 | 841 | 0.390 | 113.5 | 8.26 | 5.641 | 20,116× |
| 33 | 1,089 | 0.306 | 148.2 | 3.80 | 5.641 | 26,262× |
| 37 | 1,369 | 0.248 | 187.7 | 10.48 | 5.677 | 33,060× |
| 41 | 1,681 | 0.205 | 231.9 | 17.56 | 5.711 | 40,616× |
| 49 | 2,401 | 0.153 | 333.6 | 29.51 | 5.633 | 59,234× |

Figure 25. Operator vs. FEM accuracy and speed-up across mesh resolutions, all six cases (Tables 18/18a-e).

The anchoring effect replicates a third time: 2.99× and 2.47× at N=21/N=33 the smallest of the three B2 materials but the same shape, every time. Three of three now show it: the property of B2's two-resolution training protocol, not a coincidence of one or two materials. Whether it traces to the protocol or to the B2 geometry itself remains open no B1 checkpoint trained jointly at two resolutions exists to separate the two. Point 2 of the round-5 review is now measured for all six geometries × material combinations.

| N | Nodes | FEM rel. L2 (%) | FEM cost (s) | Operator rel. L2 (%) | Operator cost (ms) | Speed-up |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | 169 | 0.474 | 19.8 | 5.96 | 5.739 | 3,452× |
| 17 | 289 | 0.290 | 34.7 | 5.30 | 5.713 | 6,082× |
| 21 | 441 | 0.201 | 54.2 | 4.75 | 5.763 | 9,401× |
| 25 | 625 | 0.148 | 79.0 | 4.25 | 6.034 | 13,099× |
| 29 | 841 | 0.113 | 104.6 | 3.84 | 5.582 | 18,742× |
| 33 | 1,089 | 0.089 | 136.6 | 3.60 | 5.556 | 24,578× |
| 37 | 1,369 | 0.072 | 173.3 | 3.53 | 5.068 | 34,193× |
| 41 | 1,681 | 0.059 | 213.9 | 3.64 | 5.563 | 38,450× |
| 49 | 2,401 | 0.042 | 308.1 | 4.27 | 5.629 | 54,731× |

## 9. Physics-informed vs data-driven, the complete 2×2

Both losses under both optimizers, all four at 75,000 optimizer steps, B1 × Neo-Hookean, 800/200 split. Held-out relative L2.

| Training loss | Adam lr 2e-3 | AdamW lr 1e-3 + OneCycle |
| --- | --- | --- |
| Physics-informed (energy) | 0.0959 | 0.1247 |
| Data-driven (rel. L2 to FEM) | 0.1307 | 0.0826 |

Table 21. THE RANKING FLIPS. Physics-informed wins the Adam column by 27%; data-driven wins the AdamW column by 34%. Neither principle wins outright on this problem, and only the complete grid shows it either column alone supports a confident, opposite conclusion.

Figure 26. Physics-informed vs. data-driven training: validation error by loss and total cost before first inference (Tables 21/21a).

Why: the optimizer moves the two losses in OPPOSITE directions. AdamW+OneCycle makes the physics-informed model 30% worse and the data-driven model 37% better. A schedule tuned on one loss is not neutral ground for the other.

What survives the flip: the data-driven model needs 800 FEM solves = 5.65 h of CPU for labels before training starts; the physics-informed one needs none. That is a property of the principle, not of the recipe, and it does not move between columns.

Training wall-clock (Table 21): physics-informed 2,873.8s (Adam) / 3,108.9s (AdamW+OneCycle); data-driven 1,458.3s / 1,463.0s — before adding the label-generation cost above.

Total cost before the first inference (Table 21a): data-driven is more expensive by a FIXED amount — 18,924s (5.26h) under matched Adam, 18,694s (5.19h) under matched AdamW+OneCycle — for every future inference count, not a break-even threshold. Assumed equal inference cost between the two (same architecture); now confirmed directly (measured 2026-09-09): DD-NO 4.625 ms/sample vs. 4.586 ms/sample, a 0.8% difference. Under this now-confirmed equality the per-inference term cancels and the label-generation cost dominates by more than an order of magnitude in both pairings.

## 10. Verification against an analytic solution (MMS)

Everywhere else in this report FEM defines the reference, so FEM is never itself graded. A manufactured solution supplies an exact answer in closed form and scores Q4 and Q9 on equal terms. u* vanishes on the whole boundary, so homogeneous Dirichlet is exact and the shared solver needed no change; the body force b = −Div P is derived by nested autodiff and checked against a central difference. Uniform material, FP64.

| Order | N | DOF | L2 | H1 semi | Stress | Energy |
| --- | --- | --- | --- | --- | --- | --- |
| Q4 | 5 | 50 | 5.266e-02 | 2.252e-01 | 2.270e-01 | 4.959e-02 |
| Q4 | 9 | 162 | 1.351e-02 | 1.132e-01 | 1.143e-01 | 1.268e-02 |
| Q4 | 17 | 578 | 3.403e-03 | 5.666e-02 | 5.724e-02 | 3.191e-03 |
| Q9 | 5 | 162 | 3.373e-03 | 2.314e-02 | 2.355e-02 | 5.246e-04 |
| Q9 | 9 | 578 | 4.155e-04 | 5.763e-03 | 5.957e-03 | 3.327e-05 |
| Q9 | 17 | 2,178 | 5.163e-05 | 1.438e-03 | 1.495e-03 | 2.088e-06 |

Table 22. Q4 and Q9 against the manufactured solution.

| Order | Norm | Observed | Theory | Pairwise |
| --- | --- | --- | --- | --- |
| Q4 | L2 | 1.98 | 2 | 1.96, 1.99 |
| Q4 | H1 semi | 1.00 | 1 | 0.99, 1.00 |
| Q4 | Stress | 0.99 | 1 | 0.99, 1.00 |
| Q4 | Energy | 1.98 | 2 | 1.97, 1.99 |
| Q9 | L2 | 3.01 | 3 | 3.02, 3.01 |
| Q9 | H1 semi | 2.00 | 2 | 2.01, 2.00 |
| Q9 | Stress | 1.99 | 2 | 1.98, 1.99 |
| Q9 | Energy | 3.99 | 4 | 3.98, 3.99 |

Table 23. This is the verification, not decoration: a body force wrong by a sign or a factor would collapse these rates. All eight land on theory, and every pairwise rate is within 0.03 of its fit unlike Table 20's exponent, these have settled.

Q9 wins at equal cost, and its margin grows with refinement: 4.0× at 162 DOF, 8.2× at 578 DOF in L2.

The third leg is now measured. The report's trained operators cannot be pointed at a manufactured problem no body-force term in the energy functional, no input channel to carry one so a separate operator was trained for this study on the same Q4 mesh, minimizing the same discrete potential energy and scored by the same error routine as the two solvers.

Fairness point the advisor raised: since each manufactured solution generates its own body-force field b = -Div P, how does that varying forcing reach the operator -- via the family's (alpha, beta) parameters, or as a direct field? Checked in code: the operator's forcing input and FEM's own right-hand side are built by the same assembly call with identical arguments, fed to the operator directly as a per-node channel (only normalized statistically); (alpha, beta) only generate the family and are never seen by the operator itself. The operator therefore receives the same discretized load FEM does, not a lower-dimensional encoding of it.

Read the table with the ceiling in hand and read the ceiling correctly. The operator minimizes the same functional over the same Q4 space that the Q4 solver solves, so the Q4 solution is the minimizer and nothing the operator produces attains a lower Π. That is a statement about Π, and Π is none of the four errors in the table. It does not transfer to L2: a field that fails to minimize Π can still sit closer to the exact solution in L2, because Q4's discretization error is a systematic bias the network's optimization error may partly cancel. An earlier revision said a ratio below one would be a defect; that is right for Π and wrong for L2, and the three-mesh run below gives 0.37× in L2 at N = 9 with nothing defective. What the ceiling does protect, empirically, is the derivative norm: operator/Q4 is above one in H1 and in stress at all three meshes. The functional itself was verified against the solver before training ΠuFEM = -7.999050 is a true minimum, the excess grows quadratically with ratio 4.000, and a work term mis-scaled by eight moves that minimum to 0.125.

| Method | DOF | L2 | H1 semi | Stress | Energy |
| --- | --- | --- | --- | --- | --- |
| Q4 (same mesh) | 578 | 3.403e-03 | 5.666e-02 | 5.724e-02 | 3.191e-03 |
| Q9 (same N) | 2,178 | 5.163e-05 | 1.438e-03 | 1.495e-03 | 2.088e-06 |
| Physics-informed operator | 578 | 8.238e-03 | 5.831e-02 | 5.886e-02 | 9.914e-03 |

Table 24. Three-way at N = 17, α = 0.05, β = 0.7. The FEM rows are Table 22's N = 17 rows, not a second measurement. Operator: 16,000 steps, 8.2 min on an A100, 64-member family, no labels.

operator / Q4 = 2.42× in L2 at this mesh. The finding is that the four norms disagree: 1.03× in H1 and 1.03× in stress effectively at the Q4 optimum against 2.42× in L2 and 3.11× in energy. That inverts the usual ordering. The loss is built from the deformation gradient, so strain and stress are what it constrains hardest, and the displacement is pinned only through them; the same inversion appears in an independent N = 9 CPU run (1.35× against 4.71×). For a physics-informed operator an L2 displacement error overstates how wrong the mechanics are.

Tables 22-24 verify Neo-Hookean only. The same manufactured field and B1 mesh sequence were repeated for Mooney-Rivlin and Arruda-Boyce to confirm the check is not accidental to one energy functional.

| Order | N | DOF | L2 | H1 semi-norm | Stress | Energy |
| --- | --- | --- | --- | --- | --- | --- |
| Q4 | 5 | 50 | 5.286e-02 | 2.252e-01 | 2.254e-01 | 4.974e-02 |
| Q4 | 9 | 162 | 1.357e-02 | 1.132e-01 | 1.135e-01 | 1.273e-02 |
| Q4 | 17 | 578 | 3.421e-03 | 5.666e-02 | 5.686e-02 | 3.203e-03 |
| Q4 | 33 | 2,178 | 8.571e-04 | 2.834e-02 | 2.845e-02 | 8.021e-04 |
| Q9 | 5 | 162 | 3.388e-03 | 2.317e-02 | 2.298e-02 | 5.314e-04 |
| Q9 | 9 | 578 | 4.162e-04 | 5.766e-03 | 5.814e-03 | 3.386e-05 |
| Q9 | 17 | 2,178 | 5.165e-05 | 1.438e-03 | 1.459e-03 | 2.128e-06 |
| Q9 | 33 | 8,450 | 6.443e-06 | 3.593e-04 | 3.651e-04 | 1.332e-07 |

Table 22a. B1 x Mooney-Rivlin against the manufactured solution.

| Order | N | DOF | L2 | H1 semi-norm | Stress | Energy |
| --- | --- | --- | --- | --- | --- | --- |
| Q4 | 5 | 50 | 5.199e-02 | 2.251e-01 | 2.269e-01 | 4.924e-02 |
| Q4 | 9 | 162 | 1.325e-02 | 1.132e-01 | 1.142e-01 | 1.259e-02 |
| Q4 | 17 | 578 | 3.333e-03 | 5.666e-02 | 5.721e-02 | 3.166e-03 |
| Q4 | 33 | 2,178 | 8.345e-04 | 2.834e-02 | 2.862e-02 | 7.928e-04 |
| Q9 | 5 | 162 | 3.342e-03 | 2.310e-02 | 2.368e-02 | 5.529e-04 |
| Q9 | 9 | 578 | 4.144e-04 | 5.759e-03 | 5.986e-03 | 3.533e-05 |
| Q9 | 17 | 2,178 | 5.159e-05 | 1.438e-03 | 1.502e-03 | 2.223e-06 |
| Q9 | 33 | 8,450 | 6.441e-06 | 3.593e-04 | 3.758e-04 | 1.391e-07 |

Table 22b. B1 x Arruda-Boyce against the manufactured solution.

| Material | Order | Norm | Observed rate | Theory | Pairwise |
| --- | --- | --- | --- | --- | --- |
| Mooney-Rivlin | Q4 | L2 | 1.98 | 2 | 1.96, 1.99, 2.00 |
| Mooney-Rivlin | Q4 | H1 semi-norm | 1.00 | 1 | 0.99, 1.00, 1.00 |
| Mooney-Rivlin | Q4 | Stress | 1.00 | 1 | 0.99, 1.00, 1.00 |
| Mooney-Rivlin | Q4 | Energy | 1.99 | 2 | 1.97, 1.99, 2.00 |
| Mooney-Rivlin | Q9 | L2 | 3.01 | 3 | 3.03, 3.01, 3.00 |
| Mooney-Rivlin | Q9 | H1 semi-norm | 2.00 | 2 | 2.01, 2.00, 2.00 |
| Mooney-Rivlin | Q9 | Stress | 1.99 | 2 | 1.98, 1.99, 2.00 |
| Mooney-Rivlin | Q9 | Energy | 3.99 | 4 | 3.97, 3.99, 4.00 |
| Arruda-Boyce | Q4 | L2 | 1.99 | 2 | 1.97, 1.99, 2.00 |
| Arruda-Boyce | Q4 | H1 semi-norm | 1.00 | 1 | 0.99, 1.00, 1.00 |
| Arruda-Boyce | Q4 | Stress | 1.00 | 1 | 0.99, 1.00, 1.00 |
| Arruda-Boyce | Q4 | Energy | 1.99 | 2 | 1.97, 1.99, 2.00 |
| Arruda-Boyce | Q9 | L2 | 3.01 | 3 | 3.01, 3.01, 3.00 |
| Arruda-Boyce | Q9 | H1 semi-norm | 2.00 | 2 | 2.00, 2.00, 2.00 |
| Arruda-Boyce | Q9 | Stress | 1.99 | 2 | 1.98, 2.00, 2.00 |
| Arruda-Boyce | Q9 | Energy | 3.99 | 4 | 3.97, 3.99, 4.00 |

Table 23a. Observed rates for both materials, fitted the same way as Table 23. All sixteen land on theory -- the verification holds for every material this report trains an operator on, not only the one in Tables 22-24.

Richer family, correct norm (advisor, round-8 point 6, closed in the Report itself on 2026-09-15). Tables 22/22a/22b used one spatial mode (varying only two amplitudes) and reported the scalar internal-ENERGY VALUE, which superconverges at double the true rate. Tables 22c/22d/22e repeat all three materials on a genuine multi-mode field (three sine/cosine terms per displacement component) and report the ENERGY NORM instead -- the quantity Cea's lemma actually bounds. Data: point9_results/mms_richer_B1_*.json, already verified 2026-09-09; only now reflected in the documents Timon reads, not just a side file.

| Order | N | DOF | L2 | H1 semi | Stress | Energy (value) | Energy norm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Q4 | 5 | 50 | 1.256e-01 | 4.212e-01 | 3.660e-01 | 1.299e-01 | 3.789e-01 |
| Q4 | 9 | 162 | 3.173e-02 | 2.144e-01 | 1.854e-01 | 3.490e-02 | 1.906e-01 |
| Q4 | 17 | 578 | 8.016e-03 | 1.077e-01 | 9.347e-02 | 8.964e-03 | 9.588e-02 |
| Q4 | 33 | 2,178 | 2.011e-03 | 5.392e-02 | 4.685e-02 | 2.257e-03 | 4.803e-02 |
| Q9 | 5 | 162 | 1.890e-02 | 1.075e-01 | 8.381e-02 | 7.093e-03 | 8.658e-02 |
| Q9 | 9 | 578 | 2.525e-03 | 2.777e-02 | 2.118e-02 | 4.625e-04 | 2.226e-02 |
| Q9 | 17 | 2,178 | 3.197e-04 | 6.986e-03 | 5.353e-03 | 2.959e-05 | 5.611e-03 |
| Q9 | 33 | 8,450 | 4.007e-05 | 1.749e-03 | 1.342e-03 | 1.861e-06 | 1.406e-03 |

Table 22c. B1 x Neo-Hookean, richer-mode field.

| Order | N | DOF | L2 | H1 semi | Stress | Energy (value) | Energy norm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Q4 | 5 | 50 | 1.250e-01 | 4.208e-01 | 3.596e-01 | 1.266e-01 | 3.789e-01 |
| Q4 | 9 | 162 | 3.198e-02 | 2.144e-01 | 1.831e-01 | 3.509e-02 | 1.910e-01 |
| Q4 | 17 | 578 | 8.116e-03 | 1.077e-01 | 9.234e-02 | 9.042e-03 | 9.610e-02 |
| Q4 | 33 | 2,178 | 2.038e-03 | 5.392e-02 | 4.628e-02 | 2.278e-03 | 4.814e-02 |
| Q9 | 5 | 162 | 1.896e-02 | 1.076e-01 | 8.065e-02 | 7.159e-03 | 8.612e-02 |
| Q9 | 9 | 578 | 2.530e-03 | 2.778e-02 | 2.010e-02 | 4.662e-04 | 2.205e-02 |
| Q9 | 17 | 2,178 | 3.199e-04 | 6.987e-03 | 5.072e-03 | 2.984e-05 | 5.557e-03 |
| Q9 | 33 | 8,450 | 4.008e-05 | 1.749e-03 | 1.272e-03 | 1.877e-06 | 1.392e-03 |

Table 22d. B1 x Mooney-Rivlin, richer-mode field.

| Order | N | DOF | L2 | H1 semi | Stress | Energy (value) | Energy norm |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Q4 | 5 | 50 | 1.280e-01 | 4.195e-01 | 3.745e-01 | 1.293e-01 | 3.853e-01 |
| Q4 | 9 | 162 | 3.284e-02 | 2.143e-01 | 1.956e-01 | 3.708e-02 | 1.964e-01 |
| Q4 | 17 | 578 | 8.335e-03 | 1.077e-01 | 9.872e-02 | 9.551e-03 | 9.875e-02 |
| Q4 | 33 | 2,178 | 2.093e-03 | 5.392e-02 | 4.950e-02 | 2.407e-03 | 4.945e-02 |
| Q9 | 5 | 162 | 1.863e-02 | 1.073e-01 | 9.521e-02 | 8.027e-03 | 9.300e-02 |
| Q9 | 9 | 578 | 2.515e-03 | 2.775e-02 | 2.337e-02 | 5.283e-04 | 2.346e-02 |
| Q9 | 17 | 2,178 | 3.194e-04 | 6.985e-03 | 5.909e-03 | 3.387e-05 | 5.916e-03 |
| Q9 | 33 | 8,450 | 4.006e-05 | 1.749e-03 | 1.482e-03 | 2.131e-06 | 1.482e-03 |

Table 22e. B1 x Arruda-Boyce, richer-mode field.

Energy (value) is 1-2 orders tighter than every other column at matched rows -- e.g. Neo-Hookean Q4 N=33: L2 2.011e-03, H1 5.392e-02, value 2.257e-03, norm 4.803e-02 -- confirming the advisor's point: the value column alone overstates verification tightness.

| Material | Order | Norm | Observed | Theory | Pairwise |
| --- | --- | --- | --- | --- | --- |
| Neo-Hookean | Q4 | L2 | 1.99 | 2 | 1.99, 1.98, 2.00 |
| Neo-Hookean | Q4 | H1 semi-norm | 0.99 | 1 | 0.97, 0.99, 1.00 |
| Neo-Hookean | Q4 | Stress | 0.99 | 1 | 0.98, 0.99, 1.00 |
| Neo-Hookean | Q4 | Energy norm | 0.99 | 1 | 0.99, 0.99, 1.00 |
| Neo-Hookean | Q9 | L2 | 2.96 | 3 | 2.90, 2.98, 3.00 |
| Neo-Hookean | Q9 | H1 semi-norm | 1.98 | 2 | 1.95, 1.99, 2.00 |
| Neo-Hookean | Q9 | Stress | 1.99 | 2 | 1.98, 1.98, 2.00 |
| Neo-Hookean | Q9 | Energy norm | 1.98 | 2 | 1.96, 1.99, 2.00 |
| Mooney-Rivlin | Q4 | L2 | 1.98 | 2 | 1.97, 1.98, 1.99 |
| Mooney-Rivlin | Q4 | H1 semi-norm | 0.99 | 1 | 0.97, 0.99, 1.00 |
| Mooney-Rivlin | Q4 | Stress | 0.99 | 1 | 0.97, 0.99, 1.00 |
| Mooney-Rivlin | Q4 | Energy norm | 0.99 | 1 | 0.99, 0.99, 1.00 |
| Mooney-Rivlin | Q9 | L2 | 2.96 | 3 | 2.91, 2.98, 3.00 |
| Mooney-Rivlin | Q9 | H1 semi-norm | 1.98 | 2 | 1.95, 1.99, 2.00 |
| Mooney-Rivlin | Q9 | Stress | 1.99 | 2 | 2.00, 1.99, 2.00 |
| Mooney-Rivlin | Q9 | Energy norm | 1.98 | 2 | 1.97, 1.99, 2.00 |
| Arruda-Boyce | Q4 | L2 | 1.98 | 2 | 1.96, 1.98, 1.99 |
| Arruda-Boyce | Q4 | H1 semi-norm | 0.99 | 1 | 0.97, 0.99, 1.00 |
| Arruda-Boyce | Q4 | Stress | 0.97 | 1 | 0.94, 0.99, 1.00 |
| Arruda-Boyce | Q4 | Energy norm | 0.99 | 1 | 0.97, 0.99, 1.00 |
| Arruda-Boyce | Q9 | L2 | 2.96 | 3 | 2.89, 2.98, 2.99 |
| Arruda-Boyce | Q9 | H1 semi-norm | 1.98 | 2 | 1.95, 1.99, 2.00 |
| Arruda-Boyce | Q9 | Stress | 2.00 | 2 | 2.03, 1.98, 2.00 |
| Arruda-Boyce | Q9 | Energy norm | 1.99 | 2 | 1.99, 1.99, 2.00 |

Table 23b. Richer-field rates, all three materials. Energy-norm rate now matches H1's own theory (1 at Q4, 2 at Q9), not double it -- all 24 rows land on theory.

Figure 27a. Richer multi-mode family: convergence rates in the energy norm, all three materials, Q4 and Q9 (Tables 22c/22d/22e/23b).

Figure 27. Method of manufactured solutions: convergence rates, all three materials, Q4 and Q9 (Tables 22/22a/22b/23/23a).

The operator now has a convergence rate, and it is negative. Two more operators were trained, at N = 9 and N = 33, under exactly the N = 17 protocol, so the three points differ in the mesh alone.

| N | DOF | Operator L2 | Q4 L2 | op/Q4 L2 | op/Q4 H1 | op/Q4 stress |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | 162 | 5.035e-03 | 1.351e-02 | 0.37× | 1.01× | 1.01× |
| 17 | 578 | 8.238e-03 | 3.403e-03 | 2.42× | 1.03× | 1.03× |
| 33 | 2,178 | 1.136e-02 | 8.525e-04 | 13.33× | 1.36× | 1.33× |

Table 24a/24b. The operator across three meshes, same manufactured solution and same scoring routine as Tables 22–24.

Fitted in h: operator L2 -0.59 against Q4 1.99; operator H1 +0.78 against Q4 1.00. Q4's two are the control and they land on Table 23's measured 1.98 and 1.00, so the operators can be quoted beside them. The L2 error therefore gets WORSE with refinement. The reason is that the operator's error is optimization error, not discretization error: refining reduces what limits Q4 and leaves the network where it was, while enlarging the problem it has to optimize. The crossover is inside the three points at N = 9 the operator is the more accurate of the two in L2 (0.37×), by N = 33 Q4 is 13.3 times better. Two qualifications: three points over a narrow span, and the budget was held at 2,000 epochs while the problem grew from 162 to 2,178 DOF, so this is the rate at a fixed budget, not a property of the method at convergence.

What is left is optimization error, not discretization error: best held-out L2 went 1.429e-02 → 8.826e-03 over the second half of training, a further 38%, still falling slowly.

The three-way comparison is now on a family rather than one member. Every ratio in Tables 22–24b is computed on the single manufactured member α = 0.05, β = 0.7, while the operator is scored by its mean over 16 held-out members. Q4 and Q9 were therefore solved on those same 16 members, drawn with the operator run's own call, at all three meshes; the single member is not among them. Q4's observed rate comes out at 1.99 in L2 and 1.00 in H1, reproducing the 1.98 and 1.00 Table 23 measures on eight resolutions, which is what makes it a control:

| N | Q4 (family) | Q9 (family) | operator (family) | operator/Q4, family | operator/Q4, single member |
| --- | --- | --- | --- | --- | --- |
| 9 | 1.3515e-02 | 4.1550e-04 | 8.3792e-03 | 0.62× | 0.37× |
| 17 | 3.4049e-03 | 5.1628e-05 | 8.8261e-03 | 2.59× | 2.42× |
| 33 | 8.5298e-04 | 6.4423e-06 | 1.2358e-02 | 14.49× | 13.33× |

The finding survives and is sharper on the family: the operator's error grows 1.47× from N = 9 to N = 33 while Q4's falls 15.8×, for observed rates of Q4 1.99, Q9 3.01 and operator -0.28. The operator's rate is negative on each half separately, -0.07 and -0.49, so what is established is the sign everywhere rather than one exponent. Table 24's N = 17 comparison was representative 2.59× on the family against 2.42× on the member while at N = 9 the single member was materially easier and any statement there should quote the family's 0.62×, not 0.37×. A ratio below one at N = 9 is not a defect: the ceiling constrains Π, not L2.

Limits: the operator's GPU training cost is not commensurable with the CPU Newton solves and is not forced onto a common axis; Q4's spread across the family is negligible (0.2–0.3% in L2) but the operators cannot be reported, because the operator runs stored only their family mean; and the whole section still rests on one geometry and one material.

That comparison was L2 only. The same family sweep also scores H1 semi-norm, stress and energy, and they do not tell the same story:

| N | L2 | H1 semi | stress | energy |
| --- | --- | --- | --- | --- |
| 9 | 0.62× | 1.01× | 1.01× | 0.80× |
| 17 | 2.59× | 1.04× | 1.04× | 3.01× |
| 33 | 14.49× | 1.45× | 1.40× | 8.02× |

H1 and stress stay near the ceiling through the whole refinement 1.01× to 1.45× in H1, 1.01× to 1.40× in stress while L2 reaches 14.49×. Fitted rates over N = 9 to 33: the operator is 0.74 in H1 and 0.77 in stress still improving with refinement, against Q4's 1.00 but -0.28 in L2. Energy is between the two, +0.33. Per interval the direction holds throughout: +0.96 then +0.52 in H1, +0.97 then +0.57 in stress the same inversion the single-member run showed (H1 and stress protected near the variational ceiling, L2 not), now confirmed on 16 members at both refinement intervals.

A different question than the mean-ratio table above: is the operator itself as consistent from member to member as Q4 is? The same three checkpoints were re-scored per test member (no retraining) and their coefficient of variation (std/mean) compared to Q4's own, already negligible:

| N | L2 | H1 semi | stress | energy |
| --- | --- | --- | --- | --- |
| 9 | 0.384 (Q4 0.002) | 0.003 (Q4 0.000) | 0.006 (Q4 0.007) | 0.549 (Q4 0.003) |
| 17 | 0.258 (Q4 0.003) | 0.016 (Q4 0.000) | 0.015 (Q4 0.007) | 0.370 (Q4 0.003) |
| 33 | 0.425 (Q4 0.003) | 0.134 (Q4 0.000) | 0.120 (Q4 0.007) | 0.736 (Q4 0.003) |

No, in every norm at every mesh, and in L2/energy not close: the operator's std/mean is 0.258–0.425 in L2 and 0.370–0.736 in energy at every mesh against Q4's 0.000–0.007 everywhere. H1 and stress start indistinguishable from Q4's own spread at N = 9 (0.003 and 0.006) and grow away from it by N = 33 (0.134 and 0.120) the same ceiling-proximity effect the mean ratio shows, now visible in per-member reliability too.

One decision to raise: the body-force-versus-homogeneous fork was settled here, not confirmed by Timon. A body-force-free exact solution on this domain is a homogeneous deformation, which Q4 reproduces to machine precision and which would separate nothing but if he wants the study shaped differently, this is the choice to revisit.

## Error metric definition

Every reported validation/test error is a per-sample, per-component relative error:

eu=∥upred-uexact∥∥uexact∥+ε,

and analogously for ev, where

∥v∥:=1Nivi21/2

is the RMS norm of a nodal quantity over the Nmesh nodes, and ε=10-12is included to avoid division by zero. The reported “validation error” is the sample average of 0.5eu+ev.

## Final results, all six benchmark cases

Best validation error, epoch, and training time for all 6 cases (B2 rows use the corrected, loss-normalized recipe).

| Case | Best val. error | Best epoch | Final epoch | Wall-clock (s) |
| --- | --- | --- | --- | --- |
| B1 × Neo-Hookean | 0.0959 | 550 | 750 | 2873.8 |
| B1 × Mooney-Rivlin | 0.1121 | 500 | 700 | 2785.3 |
| B1 × Arruda-Boyce | 0.1070 | 475 | 675 | 2855.6 |
| B2 × Neo-Hookean | 0.0911 | 850 | 1050 | 32244.0 |
| B2 × Mooney-Rivlin | 0.0728 | 850 | 1050 | 34164.1 |
| B2 × Arruda-Boyce | 0.0981 | 525 | 725 | 24846.8 |

Table 5. Final results, all six cases

Figure 28. Training cost, per-sample inference speed-up, and training memory footprint across all six (geometry, material) cases (Tables 5/7/8).

Figure 1. Validation-error training curves, all six cases.

Figure 2. B1 × Neo-Hookean predicted vs. exact vs. error.

Figure 3. B1 × Mooney-Rivlin predicted vs. exact vs. error.

Figure 4. B1 × Arruda-Boyce predicted vs. exact vs. error.

Figure 5. B2 × Neo-Hookean predicted vs. exact vs. error.

Figure 6. B2 × Mooney-Rivlin predicted vs. exact vs. error.

Figure 7. B2 × Arruda-Boyce predicted vs. exact vs. error.

## B2 accuracy regression: root cause and fix

Boundary-force inconsistency found and fixed via loss-scale normalization; B2 now matches B1’s accuracy range.

| Recipe | Mean rel. L2 error | Note |
| --- | --- | --- |
| Original baseline (force-inconsistent) | 32.46% | Documented pre-fix baseline |
| Force-consistency fix alone | 94.08% | Regression near-trivial solution |
| Force-consistency fix + loss-scale normalization | 9.11% | Resolved best epoch 850 |

Table 13. B2 x Neo-Hookean accuracy: successive fix stages

| Case | Mean rel. L2 error | Note |
| --- | --- | --- |
| B2 × Neo-Hookean | 9.11% | lossnorm recipe, epoch 850 |
| B2 × Mooney-Rivlin | 7.28% | lossnorm recipe, trial 1/3, epoch 850; best of all six cases |
| B2 × Arruda-Boyce | 9.81% | lossnorm recipe, trial 1/3; adopted despite narrowly missing the self-imposed <9.00% search target |

Table 14. Final adopted mean relative L2 validation error for all three B2 materials, using the loss-scale-normalization fix.

Figure 29. B2 fix-attempt history (Neo-Hookean) and final adopted B2 results across materials (Tables 13/14).

Figure 11. B2 × Neo-Hookean error vs. E, ν, load.

Figure 12. B2 × Neo-Hookean worst-case sample.

Figure 13. B2 × Mooney-Rivlin error vs. E, ν, load.

Figure 14. B2 × Mooney-Rivlin worst-case sample.

Figure 15. B2 × Arruda-Boyce error vs. E, ν, load.

Figure 16. B2 × Arruda-Boyce worst-case sample.